Level 0.5: STM32F103C8T6 Memory Map

First, let's establish the real map.

The official datasheet gives the following major address regions:

STM32F103C8T6

32-bit address space

0x0000 0000

│

│  CODE / BOOT MEMORY REGION

│

▼

0x2000 0000

│

│  SRAM

│

▼

0x4000 0000

│

│  PERIPHERALS

│

▼

0xE000 0000

│

│  Cortex-M3 INTERNAL

│  PERIPHERALS

│

▼

But here's where we need to become more precise than our earlier simplified explanation.

1. Code region: 0x00000000

RM0008 says the code area starts at:

0x0000 0000

and is accessed through the Cortex-M3 ICode/DCode buses. The boot memory is mapped into this code area depending on the boot configuration.

For normal execution from internal Flash, the STM32's Flash is available through this code/boot mapping.

The official datasheet's memory map also shows Flash starting at:

0x0800 0000

with the 0x00000000 region involved in the boot-memory aliasing mechanism.

So don't memorize:

0x00000000 = Flash

as an absolute statement.

Instead:

0x00000000 is the Cortex-M code/boot region, where the STM32 can map boot memory.

That's a much more professional understanding.

2. SRAM: 0x20000000

This one is straightforward.

0x2000 0000

│

└── SRAM

RM0008 explicitly states that the SRAM data area starts at 0x20000000.

This is where things such as:

int temperature;

int counter;

char buffer[100];

can live during program execution.

And remember what we learned earlier:

Flash → program/code

SRAM  → runtime data

Stack → SRAM

For the STM32F103C8, the device has 20 KB SRAM according to ST's product information for the family.

3. Peripheral region: 0x40000000

Now things get interesting.

The official STM32F103 memory map places the peripherals beginning at:

0x4000 0000

And this region contains many different peripherals.

For example, the official map includes:

0x4000 0000   TIM2

0x4000 0400   TIM3

0x4000 0800   TIM4

...

0x4000 4400   USART2

0x4000 4800   USART3

...

0x4000 5400   I2C1

...

0x4000 6400   CAN

...

0x4000 8800   GPIOA

0x4000 8C00   GPIOB

0x4001 1000   GPIOC

...

0x4001 2800   ADC1

0x4001 2C00   ADC2

0x4001 3000   TIM1

0x4001 3400   SPI1

0x4001 3800   USART1

...

0x4002 1000   RCC

These addresses come directly from ST's memory map.

Now notice something beautiful:

Our previous hypothetical

GPIO base = 0x40010800

was not random at all.

The official STM32F103 memory map shows:

GPIOA = 0x40010800

GPIOB = 0x40010C00

GPIOC = 0x40011000

GPIOD = 0x40011400

GPIOE = 0x40011C00

So now our earlier pointer exercises are starting to become actual STM32 programming.

4. The crucial architecture

Here's the mental model I want you to develop:

STM32F103

│

▼

CPU

│

Bus system

│

┌────────────┼────────────┐

▼                         ▼                         ▼

Flash                    SRAM             Peripherals

│

┌────────┬───────────┼──────────┐

▼                ▼                         ▼                     ▼

GPIO           USART                  TIMER                ADC

The CPU doesn't need a special instruction called:

TURN_GPIO_ON

Instead, it performs a memory-mapped access to a peripheral register.

For example:

CPU

│

│ write/read address

▼

0x40010800

│

▼

GPIOA peripheral

│

▼

GPIOA register

│

▼

GPIO hardware

│

▼

PAx pin

That is the foundation of bare-metal STM32 programming.

Now let's make it concrete

We know from the official STM32F103 memory map:

GPIOA base = 0x40010800

Suppose we have a GPIOA register with offset:

0x04

Then:

Register address

=

GPIOA base + register offset

=

0x40010800 + 0x04

=

0x40010804

Exactly the calculation you've already been practicing.

And then we can represent that address in C:

volatile uint32_t *REG = (volatile uint32_t *)0x40010804;

Now:

REG

│

└── contains 0x40010804

│

▼

GPIOA

register

And:

*REG |= (1 << 5);

means:

Read register

↓

Set bit 5

↓

Write register back

We have now connected:

C → pointer → address → register → peripheral → hardware.

But we're not going to memorize addresses blindly

This is where professional embedded engineers differ from people who merely memorize code.

I want you to eventually be able to open RM0008 and independently answer:

"Why is GPIOA at 0x40010800?"

and then:

"Why is GPIOA's next register at a particular offset?"

and then:

"What does bit 5 actually control?"

and finally:

"What electrical behavior does that bit cause on PA5?"

That entire chain is our goal.

Checkpoint 0.5.1 🧠

Using the official STM32F103 memory map above:

Q1

What is the base address of:

GPIOA

Q2

What is the base address of:

GPIOB

Q3

What is the base address of:

USART1

Q4

What is the base address of:

RCC

Q5

If:

GPIOA base = 0x40010800

register offset = 0x0C

calculate the register address.

Q6

If:

GPIOA base = 0x40010800

and you create:

volatile uint32_t *REG =

(volatile uint32_t *)0x40010800;

What does:

REG

contain?

And what does:

*REG

represent?

Answers:

- 0x40010800

- 0x40010C00

- 0x40013800

- 0x40021000

- 0x4001080C

- then reg contains the address 0x40010800 and *reg gives the value present in that address

For the STM32F103, some key verified addresses are:

| Peripheral | Base address |
| --- | --- |
| GPIOA | 0x40010800 |
| GPIOB | 0x40010C00 |
| GPIOC | 0x40011000 |
| GPIOD | 0x40011400 |
| GPIOE | 0x40011800 |
| ADC1 | 0x40012400 |
| TIM1 | 0x40012C00 |
| SPI1 | 0x40013000 |
| USART1 | 0x40013800 |
| RCC | 0x40021000 |

Now let's go one level deeper

We've established:

GPIOA

↓

Base = 0x40010800

But GPIOA is not one register.

It is a peripheral containing multiple registers.

For STM32F103, we are going to learn registers such as:

GPIOA

│

├── CRL

├── CRH

├── IDR

├── ODR

├── BSRR

├── BRR

└── LCKR

Next checkpoint

Without looking it up:

If:

GPIOA base = 0x40010800

and we have:

CRL offset = 0x00

CRH offset = 0x04

IDR offset = 0x08

ODR offset = 0x0C

tell me the actual address of:

- GPIOA_CRL

- GPIOA_CRH

- GPIOA_IDR

- GPIOA_ODR

Answers:

- 0x40010800

- 0x40010804

- 0x40010808

- 0x4001080C

0.5.2: What are these registers actually doing?

Now we move from "where is the register?" to:

"What does this register control?"

This distinction is critical.

Think of GPIOA as a small hardware block:

For STM32F1, the GPIO configuration is split between CRL and CRH:

- CRL configures PA0 through PA7

- CRH configures PA8 through PA15

- IDR lets the CPU read the input state of the pins

- ODR controls/reads the output data state

These register definitions and offsets come from the STM32F1 GPIO register description in RM0008.

Here's something interesting

Suppose you want:

PA5 as a digital output.

Which register should you care about first?

PA5 is:

PA5

↓

GPIOA

↓

Pin number 5

Since PA5 is between PA0 and PA7:

PA0 ──┐

PA1   │

PA2   │

PA3   │

PA4   │

PA5   ├── CRL

PA6   │

PA7 ──┘

PA8 ──┐

PA9   │

...   ├── CRH

PA15 ─┘

Therefore:

PA5 configuration → GPIOA_CRL

And we know:

GPIOA_CRL = 0x40010800

But now comes the fun part.

Each GPIO pin gets 4 configuration bits in CRL/CRH.

For PA5:

CRL bits

31                                                        0

┌───────────────────────────┐

│ ... │ PA7 │ PA6 │ PA5 │...│

└───────────────────────────┘

↑

4 bits

Specifically, PA5 uses:

CNF5[1:0]

MODE5[1:0]

which occupy bits 20 through 23 of CRL.

So:

PA5 configuration

↓

CRL

↓

bits 23:20

This is where your earlier bit-manipulation practice becomes extremely useful.

In STM32 microcontrollers (such as the STM32F1 series), CRL and CRH are 32-bit Port Configuration Registers used to set the mode and speed of General Purpose Input/Output (GPIO) pins. [1, 2, 3]

CRL (Port Configuration Register Low)

- Controls pins: Pin 0 through Pin 7 (lower 8 pins of a GPIO port like GPIOA, GPIOB).

- Size: 32 bits total.

- Bit allocation: Each pin uses 4 bits (Bits [3:0] for Pin 0, [7:4] for Pin 1, up to [31:28] for Pin 7). [1, 2, 3]

CRH (Port Configuration Register High)

- Controls pins: Pin 8 through Pin 15 (upper 8 pins of a GPIO port).

- Size: 32 bits total.

- Bit allocation: Each pin uses 4 bits (Bits [3:0] for Pin 8, [7:4] for Pin 9, up to [31:28] for Pin 15). [1, 2]

How the 4 Bits Work for Each Pin

For every single GPIO pin, the 4 assigned bits are split into two parts:

- MODE[1:0] (Lower 2 bits): Sets the direction and maximum output speed.

  - 00: Input mode (reset state)

  - 01: Output mode, max speed 10 MHz

  - 10: Output mode, max speed 2 MHz

  - 11: Output mode, max speed 50 MHz

- CNF[1:0] (Higher 2 bits): Sets the configuration (input/output type).

  - If MODE is Input (00):

    - 00: Analog mode

    - 01: Floating input (reset state)

    - 10: Input pull-up / pull-down

    - 11: Reserved

  - If MODE is Output (01, 10, 11):

    - 00: General purpose output push-pull

    - 01: General purpose output open-drain

    - 2: Alternate function output push-pull

    - 3: Alternate function output open-drain

Checkpoint 0.5.2

Don't look up the answer yet.

Q1

PA12 belongs to:

A. CRL
B. CRH

Q2

Which 4-bit field configures PA3?

Remember each pin gets 4 bits.

Q3

Which 4-bit field configures PA10?

Q4

If PA5 uses bits 23:20, what is the starting bit number of PA5's configuration field?

Q5

If we wanted to modify only PA5's configuration, why would this be dangerous?

GPIOA_CRL = 0x00000020;

Think carefully about what happens to the other GPIO pins' configuration bits.

Answers:

Q1. PA12 → CRH

✅ Correct.

PA0–PA7   → CRL

PA8–PA15  → CRH

So PA12 is configured in CRH.

Q2. PA3 → bits 12:15

✅ Correct.

Each pin gets 4 bits:

PA0 → bits  3:0

PA1 → bits  7:4

PA2 → bits 11:8

PA3 → bits 15:12

So:

PA3 → bits 15:12

Your answer is correct. 👍

Q3. PA10 → bits 11:8

✅ Correct.

Remember, CRH starts with PA8:

PA8  →  3:0

PA9  →  7:4

PA10 → 11:8

PA11 → 15:12

...

PA15 → 31:28

So:

PA10 → bits 11:8

Q4. PA5 starts at bit 20

✅ Correct.

PA5:

PA5 → bits 23:20

Therefore its starting/lowest bit is:

20

What does GPIOA_CRL = 0x00000020 actually mean?

First, remember:

GPIOA_CRL = 0x00000020;

There are two separate things here:

1. GPIOA_CRL

This identifies which register we're accessing.

For STM32F103:

GPIOA

↓

CRL

↓

address = 0x40010800

So:

GPIOA_CRL

↓

0x40010800

2. 0x00000020

This is the 32-bit value we're writing into that register.

Convert it to binary:

0x00000020

0000 0000 0000 0000 0000 0000 0010 0000

↑

bit 5

So we're telling the hardware:

"Put 0 in every CRL bit except bit 5, which should be 1."

Now look at the CRL structure

The CRL register is 32 bits:

31                                      0

┌───────────────────────────────────────┐

│ PA7 │ PA6 │ PA5 │ PA4 │ PA3 │ PA2 │ PA1 │ PA0 │

└───────────────────────────────────────┘

Each pin gets 4 bits.

Therefore:

PA0 → bits  3:0

PA1 → bits  7:4

PA2 → bits 11:8

PA3 → bits 15:12

PA4 → bits 19:16

PA5 → bits 23:20

PA6 → bits 27:24

PA7 → bits 31:28

Now look at our value:

0x00000020

The 2 is in the second hexadecimal digit from the right:

0x 0 0 0 0 0 0 2 0

↑

That means:

0x20 = 0010 0000

↑

bit 5

And bit 5 belongs to:

PA1 → bits 7:4

So your original thought:

"0x20 relates to PA1"

was actually partially on the right track.

But saying:

"GPIOA_CRL = 0x20 accesses PA1"

isn't quite correct.

Why?

Because GPIOA_CRL accesses the entire CRL register.

The value 0x20 simply happens to put a 1 into bit 5, which is part of PA1's configuration field.

Here's the dangerous part

Imagine the CRL currently contains:

0x44444444

Break it into hexadecimal digits:

0x 4 4 4 4 4 4 4 4

│ │ │ │ │ │ │ │

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

Each 4 represents the 4-bit configuration of one pin.

Now you execute:

GPIOA_CRL = 0x00000020;

The old value:

0x44444444

is completely replaced by:

0x00000020

So:

Before:

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

4   4   4   4   4   4   4   4

After:

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

0   0   0   0   0   0   2   0

💡 This is the key!

You didn't "change PA1."

You replaced the entire CRL register, and the resulting value happens to give PA1's field the value 2.

All the other pin configurations were also replaced.

What if we only want to change PA5?

This is where masking comes in.

PA5 occupies:

bits 23:20

We want to modify those four bits without touching the other 28 bits.

Conceptually:

Before:

[PA7][PA6][PA5][PA4][PA3][PA2][PA1][PA0]

↑

change ONLY this

We don't want:

[0000][0000][new][0000][0000][0000][0000][0000]

because that would destroy the other fields.

We want:

[old ][old ][new][old ][old ][old ][old ][old ]

That is the entire purpose of read-modify-write + masks.

The golden distinction

Memorize this:

GPIOA_CRL

↓

WHOLE REGISTER

while:

bits 23:20

↓

PA5's FIELD inside that register

And:

0x00000020

↓

VALUE being written

↓

bit 5 = 1

↓

bit 5 happens to belong to PA1's field

So address/register selection, register value, and bit field selection are three different things.

Now try the quiz again

Take the original questions:

Current CRL = 0x44444444

PA5 → bits 23:20

Q1. Which hexadecimal digit represents bits 23:20?

Q2. What hexadecimal mask would contain 1111 specifically in bits 23:20 and 0 everywhere else?

Q3. Why is clearing only those four bits safer than assigning an entirely new 32-bit value to GPIOA_CRL?

Answers:

Q1. Bits 23:20 → 4 ✅

Each hexadecimal digit represents 4 bits:

Bits:  31-28 27-24 23-20 19-16 15-12 11-8  7-4  3-0

Hex:     4      4     4      4     4     4     4     4

↑

PA5

So PA5's field is represented by the third hexadecimal digit from the right.

Q2. Mask → 0x00F00000 ✅

Correct.

0x00F00000

↑

1111

Those four 1s occupy bits 23:20.

So:

0x00F00000

↓

bits 23:20 = 1111

Excellent.

Q3. Why is masking safer?

Let's use a real example.

Suppose:

Current CRL = 0x44444444

We want to change only PA5.

PA5:

bits 23:20

If we simply do:

GPIOA_CRL = new_value;

we replace all 32 bits.

For example:

Before:

0x44444444

After:

0x00000020

Everything got replaced.

That means the configurations of:

PA0

PA1

PA2

PA3

PA4

PA6

PA7

can also be changed unintentionally.

Masking lets us say:

"Touch these four bits, leave everything else exactly as it was."

Conceptually:

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

│   │   │   │   │   │   │   │

└───┴───┤   └───┴───┴───┴───┘

CHANGE

ONLY PA5

This is why embedded programmers use read-modify-write operations.

For example, the general idea is:

REG = REG & ~MASK;

First:

REG

↓

read current value

Then:

~MASK

↓

make PA5's four bits zero

Then:

&

↓

clear only PA5's field

The other bits survive unchanged.

After that, we can insert the new PA5 configuration into those four cleared bits.

Now answer this:

Suppose:

CRL = 0x44444444

MASK = 0x00F00000

We execute:

CRL = CRL & ~MASK;

What do you think the resulting CRL value will be?

The pattern is :

For  CLEAR - CRL = CRL & ~MASK;

For SET - CRL = CRL | NEW_VALUE;

0.5.3: MODE and CNF

For PA5, the four bits are:

CRL bits 23:20

23        22        21       20

┌────────┬────────┐

│  CNF5        │ MODE5   │

│  1:0            │  1:0          │

└────────┴────────┘

So:

CNF5  = bits 23:22

MODE5 = bits 21:20

This is an important correction to the simplified picture we used earlier. The four-bit field is CNF followed by MODE, from higher bits to lower bits. ST's RM0008 explicitly defines it this way.

What does MODE do?

MODE5[1:0] determines whether PA5 is an input or output, and if it is an output, the maximum output speed.

According to RM0008:

| MODE5 | Meaning |
| --- | --- |
| 00 | Input mode |
| 01 | Output, max 10 MHz |
| 10 | Output, max 2 MHz |
| 11 | Output, max 50 MHz |

So:

MODE5 = 00 → Input

MODE5 = 01 → Output 10 MHz

MODE5 = 10 → Output 2 MHz

MODE5 = 11 → Output 50 MHz

Notice something important:

MODE does not mean "make pin HIGH or LOW."

It determines the pin's operating mode and output speed.

What does CNF do?

CNF5[1:0] determines the configuration of the pin.

The meaning depends on whether MODE says we're using input or output mode.

When MODE = 00, input mode:

| CNF5 | Configuration |
| --- | --- |
| 00 | Analog input |
| 01 | Floating input |
| 10 | Input with pull-up/pull-down |
| 11 | Reserved |

When MODE ≠ 00, output mode:

| CNF5 | Configuration |
| --- | --- |
| 00 | General-purpose output push-pull |
| 01 | General-purpose output open-drain |
| 10 | Alternate-function output push-pull |
| 11 | Alternate-function output open-drain |

These are directly from the GPIO configuration table in RM0008.

Now let's configure PA5

Suppose our goal is:

PA5 = general-purpose output, push-pull, maximum speed 2 MHz.

Let's not jump to hexadecimal.

Build it from the requirements.

Step 1: Input or output?

We want output.

Therefore:

MODE5 ≠ 00

Step 2: Output speed?

We want 2 MHz.

From the table:

MODE5 = 10

Step 3: Output type?

We want:

General-purpose output push-pull

Therefore:

CNF5 = 00

So our four-bit PA5 configuration is:

CNF5 MODE5

00   10

Therefore:

0010

Now the fun part.

What hexadecimal digit is 0010?

Your turn. 🧠

Q1. CNF5 = ?

Q2. MODE5 = ?

Q3. Combined PA5 field = ???? binary.

Q4. What hexadecimal digit does that 4-bit field represent?

Q5. Since PA5 occupies bits 23:20, what value should we OR into GPIOA_CRL to configure PA5 this way?

Answers:

- 00

- 10

- 0010

- 2

- 0x00200000

Explaination for 5th answer:

So the correct mapping is:

GPIOA_CRL

31                 28 27                24 23                 20 19                 16 15                12 11                   8 7                     4 3                     0

┌──────────┬──────────┬──────────┬──────────┬──────────┬──────────┬──────────┬──────────┐

│   PA7             │   PA6               │   PA5              │   PA4                │   PA3              │   PA2              │   PA1               │   PA0             │

│   0x4              │   0x4               │   0x4              │   0x4                │   0x4              │   0x4                │   0x4              │   0x4              │

└──────────┴──────────┴──────────┴──────────┴──────────┴──────────┴──────────┴──────────┘

Therefore:

0x44444444

↑

PA5

When we clear PA5, only that third digit from the right becomes 0:

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

4   4   4   4   4   4   4   4

↓

4   4   0   4   4   4   4   4

Therefore:

0x44444444

↓

0x44044444

✅ Your earlier answer 0x44044444 was correct.

Then if we insert the desired PA5 field 0010:

PA7 PA6 PA5 PA4 PA3 PA2 PA1 PA0

4   4   2   4   4   4   4   4

we get:

0x44244444

So the complete sequence is:

0x44444444   ← initial

↓

0x44044444   ← clear PA5

↓

0x44244444   ← set PA5 = 0010

Your next challenge

Let's configure PA5 as a 50 MHz general-purpose push-pull output.

According to the table:

MODE = ?

CNF  = ?

Then answer:

- What is the 4-bit PA5 field?

- What hexadecimal digit represents it?

- What value do we OR into CRL?

- Starting from:

CRL = 0x44444444

what is the final CRL after clearing PA5's field and inserting the new configuration?

Show your calculations. This time, don't jump straight to the final hex value.

Answers:

Q1. PA5 configuration

For a 50 MHz general-purpose push-pull output:

CNF5  = 00

MODE5 = 11

So:

CNF5 MODE5

00   11

= 0011

✅ Correct

Q2. Hexadecimal

0011 = 0x3

✅ Correct

Q3. Shift into PA5's field

PA5 occupies bits 23:20.

Therefore:

0x3 << 20

gives:

0x00300000

✅ Correct

Q4. Complete CRL modification

Starting:

0x44444444

Clear PA5:

0x44044444

Insert 0011 into PA5:

0x44344444

So:

0x44444444

↓ clear PA5

0x44044444

↓ set PA5 = 0011

0x44344444

✅ Correct.

And notice the alignment:

PA7  PA6  PA5  PA4  PA3  PA2  PA1  PA0

4    4    4    4    4    4    4    4

↓

4    4    0    4    4    4    4    4

↓

4    4    3    4    4    4    4    4

So PA6 stays 4, while PA5 changes 4 → 0 → 3.

🎯 4/4.

You've now demonstrated that you can go from:

pin → register → bit field → binary → hexadecimal → register value.

That's exactly the skill we're building.

One final step before we leave this example

There's an important difference between:

GPIOA_CRL |= (0x3 << 20);

1. Compile-Time: Calculating the Bitmask (0x3 << 20)

Before the code even runs on the microcontroller, the compiler evaluates the shift operation.

- The Value: 0x3 in hexadecimal is 3 in decimal, which is written in binary as 0011.

- The Shift: << 20 tells the processor to shift those binary bits 20 positions to the left.

- The Result: The bits move to positions 21 and 20. The resulting 32-bit mask looks like this:
0000 0000 0011 0000 0000 0000 0000 0000 (or 0x00300000 in hex).

A Critical Warning About This Code

Because this code uses the OR operator (|=), it can only set bits to 1; it cannot clear bits to 0.

If bits 21 or 20 were already 1, they stay 1. However, if you are trying to change PA5 to a mode that requires a 0 in bits 22 or 23, this line will not clear them. If those bits contained old data, your pin mode will become corrupted

and:

GPIOA_CRL = (GPIOA_CRL & ~(0xF << 20)) |  (0x3 << 20);

Now:

GPIOA_CRL &= ~(0xF << 20); // Step 1: Clear all 4 configuration bits for Pin 5

GPIOA_CRL |= (0x3 << 20);  // Step 2: Safely set the new mode bits

The first does not guarantee that PA5 becomes 0011, because its existing four bits might contain 0101, 0110, etc.

The second explicitly:

- Clears PA5's entire 4-bit field.

- Inserts 0011.

- Preserves every other GPIO configuration.

This pattern is extremely important:

REG = (REG & ~MASK) | VALUE;

Think of it as:

Erase the old field, then install the new field, without disturbing the neighbors.

Next, we're going to connect this to the actual hardware behavior.

We'll take:

PA5

↓

GPIOA_CRL

↓

MODE5 = 11

CNF5  = 00

and understand what "50 MHz output, push-pull" physically means.

Then we'll answer the big question:

Does setting MODE5 = 11 actually make PA5 HIGH?

Don't answer yet. Think about it first. This is a very important distinction between pin configuration and pin output state.

The answer is:

No. MODE5 = 11 does NOT make PA5 HIGH.

This is a very important distinction.

Think of PA5 as having two separate controls

1. Configuration

CRL decides what PA5 is and how it behaves.

For example:

MODE5 = 11

CNF5  = 00

means:

PA5

↓

General-purpose output

Push-pull

Maximum output speed = 50 MHz

It essentially says:

"PA5 is allowed to operate as a push-pull output."

But it doesn't say whether the output is currently HIGH or LOW.

2. Output state

A different register, called ODR (Output Data Register), determines the output data state.

For PA5:

GPIOA_ODR

↓

bit 5

↓

PA5 output state

Conceptually:

ODR bit 5 = 0  → PA5 LOW

ODR bit 5 = 1  → PA5 HIGH

So the complete picture is:

GPIOA

│

┌───────┴────────┐

│                                     │

CRL                              ODR

│                                   │

│                                   └── "What output value?"

│                                     │

│                                  0 / 1

│

└── "How should PA5 operate?"

│

MODE + CNF

Example

First configure PA5:

MODE5 = 11

CNF5  = 00

Now PA5 is a 50 MHz general-purpose push-pull output.

But if:

ODR bit 5 = 0

then:

PA5 = LOW

If later:

ODR bit 5 = 1

then:

PA5 = HIGH

The distinction to memorize

CRL configures the pin. ODR controls the output state.

This distinction is fundamental in STM32 GPIO programming.

And don't worry about the 50 MHz part yet. "50 MHz" here is an output-speed configuration, not "PA5 is producing a 50 MHz signal." We'll unpack that carefully later.

Now I want you to answer just one simple question:

If PA5 is already configured as a general-purpose push-pull output, which register would you change to make PA5 HIGH: CRL or ODR?

Ans: ODR

For PA5, we had:

CRL:

CNF5  MODE5

00    11

So the configuration field is:

0011

That stays 0011 whether PA5 is LOW or HIGH.

Then ODR separately contains the output state:

ODR bit 5 = 0 → LOW

ODR bit 5 = 1 → HIGH

So don't combine them into 10011.

Think of it as two separate registers

GPIOA_CRL

│

└── PA5 configuration = 0011

│

└── "I am a push-pull output at 50 MHz"

GPIOA_ODR

│

└── PA5 state = 1

│

└── "Currently output HIGH"

So:

PA5 configured:

0011

PA5 HIGH:

ODR bit 5 = 1

Not:

10011

This separation is actually a very important embedded concept:

Configuration says what the pin is. Data says what state the pin outputs.

GPIOx_ODR: Output Data Register

For GPIOA:

GPIOA base = 0x40010800

ODR offset = 0x0C

Therefore:

GPIOA_ODR = 0x4001080C

The important part is what the bits mean.

GPIOA_ODR

Bit:  31 ........ 6   5   4   3   2   1   0

│   │   │   │   │   │   │

PA5 PA4 PA3 PA2 PA1 PA0

So:

ODR bit 0 → PA0

ODR bit 1 → PA1

ODR bit 2 → PA2

ODR bit 3 → PA3

ODR bit 4 → PA4

ODR bit 5 → PA5

...

ODR bit 15 → PA15

For PA5:

GPIOA_ODR |= (1 << 5);

means:

1 << 5

↓

0x20

So if ODR was initially zero:

Before:

GPIOA_ODR = 0x00000000

Set bit 5:

GPIOA_ODR |= 0x00000020

After:

GPIOA_ODR = 0x00000020

And PA5 becomes HIGH.

The key comparison

CRL

PA5 → bits 23:20

→ configuration

ODR

PA5 → bit 5

→ output data

So don't carry the CRL bit position into ODR. The same physical pin can have completely different bit positions in different registers.

For example:

PA5

│

├── CRL bits 23:20 → MODE/CNF

│

└── ODR bit 5      → output state

ODR vs IDR

ODR → "What output data am I driving?"

IDR → "What logic level is actually present at the pin?"

So IDR is primarily the register you use to read the input state of the GPIO pin, while ODR is used to set/read the output data register value.

For example:

GPIOA

│

┌───────┴───────┐

│                                   │

ODR                           IDR

│                                 │

output data       input state

│                                │

▼                             ▲

PA5 ─────────────┘

This distinction becomes especially useful when we connect a push button + LED.

Quick checkpoint

If PA5 is configured as an output and:

GPIOA_ODR = 0x00000020

then:

- Which ODR bit is 1?

- Is PA5 being driven HIGH or LOW?

- What hexadecimal value would you write to ODR to clear only PA5, while preserving all other output bits?

Answers:

Q1

0x20 means:

0000 0000 0000 0000 0000 0000 0010 0000

↑

bit 5

So bit 5 = 1. ✅

Q2

Because ODR bit 5 corresponds to PA5:

ODR bit 5 = 1

↓

PA5 = HIGH

✅ Correct.

Q3. Clear only PA5

We want to clear bit 5, while leaving every other bit unchanged.

The bit mask for PA5 is:

1 << 5 = 0x20

To clear it, we use:

GPIOA_ODR &= ~(1 << 5);

or:

GPIOA_ODR &= ~0x20;

The important part is:

~0x20

Let's expand it:

0x20 = 00000000 00000000 00000000 00100000

~0x20 = 11111111 11111111 11111111 11011111

Now notice:

bit 5

↓

~0x20 = 11111111 11111111 11111111 11011111

↑

becomes 0

When we AND the register with this:

REG & 11111111 11111111 11111111 11011111

bit 5 is forced to 0, while all the other bits remain unchanged.

So the answer is:

GPIOA_ODR &= ~(1 << 5);

Not simply:

GPIOA_ODR = 0x00000000;

because that would clear every output bit, potentially affecting PA0, PA1, PA2, etc.