0.5.4: IDR vs ODR

We've already established:

ODR → Output Data Register

IDR → Input Data Register

The important idea is that IDR lets the CPU read the logic level present on the GPIO pins.

For GPIOA:

GPIOA_IDR = 0x40010808

Each pin corresponds to the same bit number:

IDR bit 0  → PA0

IDR bit 1  → PA1

IDR bit 2  → PA2

...

IDR bit 5  → PA5

...

IDR bit 15 → PA15

Notice the beautiful symmetry:

PA5

│

┌─────┴─────┐

│                          │

ODR                   IDR

bit 5                    bit 5

│                         │

▼                      ▲

drive               read pin

So for PA5:

ODR bit 5 = 1

↓

request/output PA5 HIGH

while:

IDR bit 5 = 1

↓

the pin is being read as HIGH

These are related, but not conceptually the same thing.

Example

Suppose:

GPIOA_IDR = 0x00000020

Convert:

0x20 = 0010 0000

Therefore:

IDR bit 5 = 1

So the GPIO hardware is reporting:

PA5 = HIGH

To test PA5 specifically, we can use:

if (GPIOA_IDR & (1 << 5))

{

// PA5 is HIGH

}

Why does this work?

GPIOA_IDR

&

0x00000020

If bit 5 is 1:

0x20 & 0x20 = 0x20

Non-zero → condition is true.

If bit 5 is 0:

0x00 & 0x20 = 0x00

Zero → condition is false.

Now let's connect all three registers

Imagine we want PA5 to be an output.

Step 1: CRL

Configure PA5:

CNF5 = 00

MODE5 = 11

Therefore:

PA5 = general-purpose push-pull output

maximum output speed = 50 MHz

Step 2: ODR

GPIOA_ODR |= (1 << 5);

Now we're commanding PA5 HIGH.

Step 3: IDR

GPIOA_IDR & (1 << 5)

Now we're reading the pin's input state.

So the overall picture becomes:

GPIOA

│

┌───────┼───────┐

│                │                 │

CRL         ODR            IDR

│               │                  │

│               │                  └── Read pin

│               │

│               └────────── Drive output

│

└────────────────── Configure pin

This is the foundation for things like:

Button → IDR → CPU → ODR → LED

The CPU reads a button state and then decides what to do with an LED.

Checkpoint 0.5.4 🧠

Suppose:

GPIOA_IDR = 0x00000028

Remember:

0x28 = 0x20 + 0x08

Q1

Which two bits are 1?

Q2

Which two GPIO pins correspond to those bits?

Q3

What will this expression produce?

GPIOA_IDR & (1 << 5)

Q4

What will this expression produce?

GPIOA_IDR & (1 << 2)

Q5

Based on the IDR value, is PA5 HIGH or LOW?

Take them one at a time. This will reinforce the exact connection between hex → bits → GPIO pins → C expression.

Answers:

Let's make that distinction very clear.

Given:

GPIOA_IDR = 0x00000028

Convert only the interesting part:

0x28 = 0010 1000

Number the bits:

Bit:   7 6 5 4 3 2 1 0

0 0 1 0 1 0 0 0

↑   ↑

5   3

Therefore:

Bit 5 = 1 → PA5 HIGH

Bit 3 = 1 → PA3 HIGH

So your first two answers are correct. ✅

Q3

GPIOA_IDR & (1 << 5)

We know:

1 << 5 = 0x20

Therefore:

0x28 & 0x20

Binary:

0010 1000

& 0010 0000

-----------

0010 0000

Result:

0x20

✅ Correct.

Q4

GPIOA_IDR & (1 << 2)

First:

1 << 2 = 0x04

Then:

0x28 & 0x04

0010 1000

& 0000 0100

-----------

0000 0000

Wait! Here's the important correction:

The correct result is 0x00, not 0x04.

Because GPIOA_IDR = 0x28 has bits 5 and 3 set. Bit 2 is 0.

So:

GPIOA_IDR & (1 << 2)

= 0x28 & 0x04

= 0x00

Therefore Q4 was incorrect, but your Q1, Q2, Q3, and Q5 were correct.

Q5

Since bit 5 is 1:

PA5 = HIGH

✅ Correct.

And this is probably the exact confusion you mentioned

You seem to have thought:

"1 << 2 gives 0x04, therefore the expression gives 0x04."

But there's a second operation:

GPIOA_IDR & 0x04

The mask tells us which bit to inspect. The actual register value determines whether that bit is 0 or 1.

Think:

(1 << 2) = 0x04

means:

"I want to inspect bit 2."

Then:

IDR & 0x04

asks:

"Is bit 2 actually set?"

For our IDR:

0x28 = 0010 1000

↑

bit 3

↑

bit 5

Bit 2 is not set, so the answer is 0x00.

Very useful rule

1 << N

↓

creates a mask for bit N

REG & (1 << N)

↓

checks whether bit N is set

If the bit is set:

→ non-zero

If the bit is clear:

→ 0

Let's lock in the three GPIO registers before moving on:

CRL → configure PA0–PA7

CRH → configure PA8–PA15

ODR → control output data

IDR → read pin state

For PA5:

CRL bits 23:20 → MODE5 + CNF5

ODR bit 5      → output data

IDR bit 5      → read pin state

And the bit-mask rule:

Set bit     → REG |=  (1 << N)

Clear bit   → REG &= ~(1 << N)

Check bit   → REG &   (1 << N)

You've now got the essential register + bit manipulation foundation for GPIO.

Next step

We'll take everything we've learned and build the complete logic for:

STM32F103

↓

GPIOA

↓

PA5

↓

Configure PA5 as output

↓

Set PA5 HIGH

↓

Set PA5 LOW

Lesson 0.5.5: The missing piece, RCC

So far you know:

CPU

↓

GPIOA

↓

CRL  → configure PA5

↓

ODR  → control PA5

But there is one important piece missing:

CPU

↓

RCC

↓

Enable clock for GPIOA

↓

GPIOA

↓

CRL

↓

PA5

Why do we need RCC?

Think of a peripheral as a machine that can be powered/clocked into operation.

The STM32 does not necessarily keep every peripheral clock running all the time. The RCC, Reset and Clock Control, manages peripheral clocks.

For GPIOA, we need to tell RCC:

"Enable the clock for GPIOA."

The official RM0008 specifies:

RCC base address       = 0x40021000

RCC_APB2ENR offset     = 0x18

Therefore:

RCC_APB2ENR address

= 0x40021000 + 0x18

= 0x40021018

And:

IOPAEN = bit 2

So:

RCC_APB2ENR |= (1 << 2);

means:

Set bit 2

↓

IOPAEN = 1

↓

GPIOA clock enabled

This is confirmed directly in RM0008.

🧠 Your turn

Don't write code yet.

Imagine these registers initially contain:

RCC_APB2ENR = 0x00000000

We execute:

RCC_APB2ENR |= (1 << 2);

Answer these 4:

Q1. What is (1 << 2) in hexadecimal?

Q2. What will RCC_APB2ENR become?

Q3. Which bit of RCC_APB2ENR changed?

Q4. After this operation, what does that bit mean for GPIOA?

Answers:

- 0x04

- 0x00000004

- 3rd bit from right side

- the clock will be enabled

Now let's connect the whole chain

We're going to make PA5 an output.

Our hardware path becomes:

STM32F103

│

▼

RCC

│

APB2 peripheral bus

│

GPIOA clock enabled

│

▼

GPIOA

┌───────┐

│  CRL       │ ← Configure PA5

│  ODR     │ ← Control PA5

│  IDR      │  ← Read PA5

└──────-┘

│

▼

PA5

│

▼

LED + resistor

This is an important transition.

Previously, we were learning:

"What does this register mean?"

Now we're learning:

"How do multiple registers cooperate to make actual hardware do something?"

That's much closer to real bare-metal engineering. 🛠️

Your next challenge

We need to configure PA5 as a general-purpose push-pull output.

From what you've already learned:

GPIOA_CRL = 0x40010800

For PA5:

MODE5 = bits 21:20

CNF5  = bits 23:22

We want:

MODE5 = 11

CNF5  = 00

Therefore the 4-bit PA5 field is:

CNF5 MODE5

00    11

= 0011

= 0x3

Don't calculate the final register yet.

Answer these:

Q1. PA5 occupies which four bits of GPIOA_CRL?

Q2. If we want to clear only PA5's configuration field, what hexadecimal mask would you use?

Q3. After clearing PA5's field in:

GPIOA_CRL = 0x44444444

what should the register become?

Q4. Where would the 0x3 for PA5's configuration need to be shifted?

Answers:

- 23:20

- 0x00300000

- 0x44044444

- (0x3 << 20)

Q1. PA5 field

You said:

23:20

✅ Correct.

GPIOA_CRL

31                  24 23    20 19    16 ... 7     4 3     0

┌────────────────────┬────────┬────────┬─────...────┬───────┐

│       PA7          │  PA5   │  PA4   │      ...   │  PA1  │ PA0

└────────────────────┴────────┴────────┴─────...────┴───────┘

↑

bits 23:20

More precisely, each pin gets a 4-bit field:

PA5:

bits 23:22 = CNF5

bits 21:20 = MODE5

Q2. Small correction ⚠️

You answered:

0x00300000

That is the value for MODE5 = 11, not the mask for the entire PA5 field.

The complete PA5 field occupies bits 23:20, so the mask is:

0000 0000 1111 0000 0000 0000 0000 0000

↑

bits 23:20

= 0x00F00000

So:

PA5 field mask = 0x00F00000

This distinction is important:

0x00F00000  → mask for PA5's entire 4-bit field

0x00300000  → value representing MODE5 = 11

Good catch opportunity here. This is exactly why we're doing the calculations instead of memorizing register code. 😎

Q3. Clear PA5 field

You said:

0x44044444

✅ Correct.

Starting:

GPIOA_CRL = 0x44444444

Clear bits 23:20:

0x44444444

&

0xFF0FFFFF

──────────

0x44044444

Notice what happened:

0x4 4 0 4 4 4 4 4

↑

PA5 field

Only the PA5 field became zero.

PA6 remains 4. PA4 remains 4.

Q4. Shift the configuration

You said:

(0x3 << 20)

✅ Exactly.

Because PA5's 4-bit field begins at bit 20:

0x3 << 20

produces:

0x00300000

Then:

0x44044444

|

0x00300000

──────────

0x44344444

Therefore:

GPIOA_CRL = 0x44344444

for:

PA5:

MODE5 = 11

CNF5  = 00

which means general-purpose output push-pull, maximum output speed 50 MHz.

One important terminology point: 50 MHz here is the GPIO output-speed configuration, not saying your LED will blink at 50 MHz. The actual signal frequency depends on how you drive the pin.

🧩 Now let's build the actual register operation

Instead of directly writing:

GPIOA_CRL = 0x44344444;

a professional bare-metal programmer usually wants to modify only PA5's field without accidentally disturbing PA0-PA4 or PA6-PA7.

The pattern is:

GPIOA_CRL = (GPIOA_CRL & ~MASK) | VALUE;

For PA5:

GPIOA_CRL = (GPIOA_CRL & ~0x00F00000)  | (0x3 << 20);

Conceptually:

READ CRL

↓

CLEAR PA5's 4 bits

↓

INSERT desired PA5 configuration

↓

WRITE CRL

This pattern will follow you everywhere in embedded programming:

read → mask → modify → write.

🎯 Final challenge before we write the LED program

Suppose:

GPIOA_CRL = 0x44444444

and we execute:

GPIOA_CRL =   (GPIOA_CRL & ~0x00F00000) | (0x3 << 20);

Tell me:

1. What is ~0x00F00000?

2. What is the result of:

0x44444444 & ~0x00F00000

3. What is:

0x3 << 20

4. What is the final GPIOA_CRL?

Once you nail this, we'll write our first complete STM32F103 bare-metal GPIO program.

Answers:

- 0xFF0FFFFF

- 0x44044444

- set the 20:23 bits to 0011

- 0x44344444

Now we build the actual LED circuit

We're ready for the first real bare-metal sequence.

Our program needs to do this:

1. Enable GPIOA clock

↓

2. Configure PA5 as output

↓

3. Set PA5 HIGH

↓

4. LED turns ON

There is one more important concept before I give you the code:

Why can't we just do this?

GPIOA_ODR |= (1 << 5);

What would happen if we never enabled GPIOA's clock first?

Think about the hardware path:

CPU

↓

RCC

↓

GPIOA clock

↓

GPIOA peripheral

↓

ODR

↓

PA5

Your next checkpoint 🧠

Q1. If IOPAEN = 0, is GPIOA's peripheral clock enabled?

Q2. If GPIOA's clock is disabled, what problem would you expect when trying to configure/use GPIOA?

Q3. Put these operations in the correct order:

A. Set PA5 HIGH

B. Enable GPIOA clock

C. Configure PA5 as output

Give me Q1, Q2, Q3, and then we'll write the complete register-level code together,

Answers:

- no

- The GPIOA peripheral clock is disabled, so the GPIOA peripheral is not clocked and cannot be used normally.

- B,C,A