0.12 C → Assembly → Hardware

Take a simple statement:

GPIOA->ODR |= (1 << 5);

You already know what this means conceptually:

Set bit 5 of the GPIOA output data register.

But the STM32 CPU doesn't understand C.

The Cortex-M3 executes machine instructions.

So there is a transformation pipeline.

1. C source

You write:

GPIOA->ODR |= (1 << 5);

This is human-friendly programming language.

↓

2. Compiler

The compiler translates the C program into assembly instructions appropriate for the target CPU.

Conceptually:

C statement

↓

ARM/Thumb assembly

The exact assembly depends on the compiler, optimization settings, and surrounding code, so we should not memorize a particular instruction sequence.

↓

3. Assembler

The assembler converts assembly instructions into machine-code representation that the processor can execute.

↓

4. Linker

Your project isn't usually just one source file.

You may have:

main.c

startup file

HAL files

other .c files

libraries

The linker combines the compiled pieces and determines where code and data belong in memory.

For example:

Flash

0x08000000

│

├── Vector table

├── Program code

└── Constants

SRAM

0x20000000

│

├── .data

├── .bss

├── heap

└── stack

↓

5. Executable image

The build system produces an executable image such as an ELF file.

It can also generate formats such as HEX for programming the MCU.

↓

6. Flash

The program is programmed into the STM32's internal Flash.

After reset, the Cortex-M3 starts executing instructions from the programmed image.

↓

7. CPU executes instructions

The Cortex-M3 fetches instructions from memory and executes them.

Eventually it reaches the instructions corresponding to:

GPIOA->ODR |= (1 << 5);

↓

8. Memory-mapped peripheral access

This is where our earlier memory-mapped I/O concept becomes extremely important.

GPIOA's ODR has a specific address.

The CPU performs a memory access to that address.

The bus/interconnect routes that access to the GPIO peripheral.

↓

9. GPIO hardware changes

The GPIO peripheral updates the corresponding output bit.

For PA5:

ODR bit 5 = 1

↓

GPIO output logic

↓

PA5 output becomes HIGH

Now software has caused a physical electrical change on the MCU pin.

The complete picture

YOU

│

▼

C CODE

│

▼

COMPILER

│

▼

ASSEMBLY CODE

│

▼

ASSEMBLER

│

▼

MACHINE CODE

│

▼

LINKER

│

▼

ELF / HEX

│

▼

STM32 FLASH

│

▼

CORTEX-M3

│

▼

BUS

│

▼

GPIO REGISTER

│

▼

GPIO HARDWARE

│

▼

PA5 PIN

│

▼

PHYSICAL SIGNAL

That's the software → silicon → electricity chain.

Three things I want you to remember

① C is not what the CPU executes

C → compiler → machine instructions → CPU

② Registers are the bridge

Your C code can control hardware because peripheral registers are memory-mapped.

C code

↓

register access

↓

bus

↓

peripheral hardware

③ The build process matters

When you eventually start writing bare-metal STM32 code, understanding this pipeline will help you understand:

- startup files

- linker scripts

- vector tables

- .text

- .data

- .bss

- Flash/SRAM placement

- compiler output

- debugging

You don't need to master those yet. We will encounter them naturally at the appropriate level.

Final Level 0 checkpoint

Answer these three:

Q1. The Cortex-M3 cannot directly execute C code. What does it actually execute?

Q2. Why can a C statement modify a hardware peripheral register?

Q3. Put these in the correct order:

Linker

C source

Machine code

Compiler

Flash

CPU

Once you answer these, we'll officially mark LEVEL 0 COMPLETE and move into Level 1, where things start getting much more hands-on. 🚀

Answers:

- C → compiler → machine instructions → CPU

- because they are memory mapped

- c source, compiler, machine code, linker, CPU, flash

Perfect. 3/3 correct. ✅

One small ordering correction for Q3:

You gave:

C source → Compiler → Machine code → Linker → CPU → Flash

The conceptual build/programming flow is better represented as:

C source

↓

Compiler

↓

Machine code / object code

↓

Linker

↓

Executable image

↓

Flash

↓

CPU

So you had the right components, but Flash comes before CPU execution.

Q1

Correct. The Cortex-M3 ultimately executes machine instructions.

Q2

Correct. Because peripheral registers are memory-mapped, normal load/store operations can access hardware registers.

Q3

Almost perfect. Just remember:

Link → program into Flash → CPU fetches and executes.

🎓 LEVEL 0 COMPLETE

You've now covered the complete foundation:

0.1  MCU Fundamentals              ✅

0.2  ARM Ecosystem                 ✅

0.3  STM32 Ecosystem               ✅

0.4  Cortex-M3 Architecture        ✅

0.5  Memory                        ✅

0.6  Registers & Bit Manipulation  ✅

0.7  Programmer's Model            ✅

0.8  Buses                         ✅

0.9  Clock & Reset                 ✅

0.10 Interrupt Fundamentals       ✅

0.11 Startup Sequence              ✅

0.12 C → Assembly → Hardware       ✅

Your Level 0 mental model should now be:

And the software side:

C

↓

Compiler

↓

Machine instructions

↓

Flash

↓

Cortex-M3

↓

Bus

↓

Peripheral register

↓

Hardware

This is the foundation we'll build everything else on.