Level 0.2: ARM → Cortex-M → STM32

1.What is ARM?

You answered:

Advanced RISC Machine

That's close enough for our purposes. More importantly, ARM is an architecture/company ecosystem, not a microcontroller family.

ARM develops and licenses processor architectures and CPU designs.

Think of ARM as the organization behind a particular way of designing processors.

The key idea is:

ARM

│

├── Instruction Set Architectures

│

└── Processor IP / CPU cores

Companies such as STMicroelectronics can build their own chips around licensed ARM Cortex cores.

So:

STMicroelectronics does not create the Cortex-M3 CPU core from scratch.

They take an ARM Cortex-M3 core and build their STM32 microcontroller around it.

2. What is Cortex?

You said:

"Cortex is an ARM family."

You're very close.

Cortex is the name ARM uses for a family of processor cores.

Think:

ARM

│

└── Cortex

│

├── Cortex-A

├── Cortex-R

└── Cortex-M

But these three branches have very different purposes.

3. Cortex-A

Cortex-A = Application processors

These are designed for relatively powerful systems that can run operating systems such as Linux or Android.

Conceptually:

Cortex-A

↓

Powerful application processor

↓

Linux / Android

↓

Smartphone / SBC / computing system

For example, smartphones commonly use ARM-based application processors.

You don't need to focus heavily on Cortex-A for our STM32 journey.

4. Cortex-R

Cortex-R = Real-time processors

They're designed for applications requiring deterministic, real-time behavior and high reliability.

Think:

Cortex-R

↓

Real-time processing

↓

Hard real-time systems

You'll encounter these in areas such as automotive and storage/control applications.

Again, we'll mostly leave this branch alone for now.

5. Cortex-M

Cortex-M is ARM's family of processor cores designed primarily for microcontroller and embedded applications.

So your answer:

"Cortex M means family of series M"

was heading in the right direction.

A better mental model is:

ARM

↓

Cortex

↓

Cortex-M

↓

Different Cortex-M CPU cores

For example:

Cortex-M0

Cortex-M0+

Cortex-M3

Cortex-M4

Cortex-M7

Cortex-M33

6. So what is the "M3"?

You said:

"M3 I don't know."

Perfect. This is the important part.

M3 identifies a particular Cortex-M processor core architecture.

So:

Cortex-M

│

├── M0

├── M0+

├── M3

├── M4

├── M7

└── M33

They're different generations/designs of Cortex-M cores with different capabilities.

For our first STM32:

STM32F103

↓

Cortex-M3

7. Now let's fix your answer to Q5

You said:

"F103 means Cortex M3 family."

Not quite.

This is an important correction.

F103 does not mean Cortex-M3.

Instead:

STM32F103

│

├── STM32 → STMicroelectronics MCU family

│

└── F1 → STM32 product series

And that particular STM32F103 series uses a Cortex-M3 CPU core.

So the relationship is:

ARM

│

└── Cortex

│

└── Cortex-M

│

└── Cortex-M3

│

│ used inside

↓

STM32F103

│

│ mounted on

↓

Blue Pill

This distinction is extremely important.

8. Let's make the hierarchy crystal clear

Suppose you have a Blue Pill containing:

STM32F103C8T6

Break the relationship down:

So there are three different levels here:

Cortex-M3

The CPU core.

STM32F103C8T6

The complete microcontroller containing that CPU core plus memory and peripherals.

Blue Pill

The development board containing the MCU and supporting circuitry.

9. Now compare it with your future STM32F4

Suppose you later buy an STM32F407 board.

The relationship becomes:

ARM

↓

Cortex

↓

Cortex-M

↓

Cortex-M4

↓

STM32F407 MCU

↓

STM32F407 development board

Your uploaded document gives this exact F103 → M3 and F407 → M4 relationship.

This is why I don't want you to think:

"I'm learning STM32F103."

Instead:

"I'm using STM32F103 as my first platform for learning Cortex-M embedded systems."

That's a much better way to think.

One more distinction: Architecture vs Core vs MCU

This will save you a lot of confusion later.

Think of these as different layers:

ARM architecture

↓

Cortex-M processor family

↓

Specific CPU core

↓

STM32 microcontroller

↓

Development board

For example:

ARM

↓

Cortex-M

↓

Cortex-M3

↓

STM32F103C8T6

↓

Blue Pill

And:

ARM

↓

Cortex-M

↓

Cortex-M4

↓

STM32F407

↓

STM32F4 development board

Why are we learning this?

Because later, when someone says:

"This is an STM32F407 Cortex-M4 MCU."

you should immediately understand:

STM32F407

│

├── MCU made by ST

│

└── contains

↓

Cortex-M4 CPU core

And if someone says:

"I know Cortex-M4."

you should not assume they know STM32.

Cortex-M4 knowledge can transfer to many manufacturers:

ARM Cortex-M4

│

├── STM32

├── NXP

├── TI

├── Microchip

└── Nordic

The surrounding peripherals and implementation differ, but the Cortex-M foundation transfers.

Checkpoint:

"What is the difference between ARM, Cortex-M, Cortex-M3, STM32F103C8T6, and the Blue Pill?"

My answer:

arm is the architecture rented from their company to use in some other company boards like stm, nxp etc, cortex m is the kind of processors designed for microcontrollers and for embedded systems,cortex m3 is the 3 generation of processor which is fast and better than previous generations, stm32f103c8t6 is the mcu and bluepill is development board made by this muc and m3 arm processor

Your answer, refined

ARM provides processor architectures and processor IP that companies such as STMicroelectronics and NXP can license and build into their own chips.

Your "rented" analogy is useful for remembering the idea, but in technical discussions say licensed, not rented.

Cortex-M is a family of ARM processor cores designed for microcontroller and embedded applications.

Correct.

Cortex-M3 is a specific Cortex-M processor core.

Small correction: don't call it the "3rd generation." M3 is the name of that particular Cortex-M core/profile, not simply "generation 3." Comparing M3 and M4 as "M4 is just a newer generation and therefore faster" can lead you astray because their capabilities and intended features differ.

STM32F103C8T6 is the complete MCU.

Correct.

Blue Pill is a development board built around the STM32F103C8T6 MCU.

Correct. And the MCU contains the Cortex-M3 core.

So the clean professional explanation is: