0.11 Startup Sequence

Now we connect the Cortex-M3 hardware to the C program you write.

When you write:

int main(void)

{

// your code

}

the CPU does not magically start at main().

There is a sequence that happens first.

1. What happens after reset?

For an STM32F103, think of the simplified sequence as:

RESET

│

▼

┌─────────────────────┐

│ CPU reads Vector    │

│ Table from Flash    │

└──────────┬──────────┘

│

┌────────┴────────┐

▼                 ▼

Initial Stack Pointer    Reset_Handler

│                 │

│                 ▼

│          Startup code

│                 │

│                 ▼

│          System initialization

│                 │

└────────┐        │

▼        ▼

main()

There are two particularly important things the CPU obtains from the vector table immediately after reset:

- Initial Stack Pointer

- Reset Handler address

2. Where is the vector table?

For the normal STM32 startup configuration, the vector table is located at the beginning of the code memory region, typically:

0x08000000

for STM32F1 internal Flash.

Conceptually:

Flash

0x08000000 ┌─────────────────────────┐

│ Initial Stack Pointer   │

├─────────────────────────┤

0x08000004 │ Reset_Handler address  │

├─────────────────────────┤

0x08000008 │ NMI_Handler address    │

├─────────────────────────┤

0x0800000C │ HardFault_Handler      │

├─────────────────────────┤

│ ...                     │

└─────────────────────────┘

The vector table is essentially a table of addresses associated with reset and exceptions/interrupts.

We will study the interrupt entries more deeply later. For now, concentrate on the first two entries.

3. Initial Stack Pointer

Remember our earlier discussion of SP?

The Cortex-M has a stack pointer, and after reset the processor needs to know where the stack should begin.

So the first word of the vector table contains the initial stack pointer value.

For example, conceptually:

0x08000000 → 0x20005000

That means:

Initial SP = 0x20005000

Notice the important distinction:

0x08000000

↑

address where the vector-table entry is stored

0x20005000

↑

value stored in that entry

= initial stack pointer

The stack itself is in SRAM, whose address range begins at 0x20000000 on this STM32 family.

4. Reset Handler

The second vector-table entry contains the address of the Reset Handler.

Conceptually:

0x08000004 → 0x080001XX

So after reset, the CPU obtains the Reset Handler address and begins execution there.

Think:

Vector Table

│

├── Initial SP

│

└── Reset_Handler address

│

▼

Reset_Handler()

The Reset Handler is normally provided by the startup code generated for the STM32 project.

5. What does Reset_Handler do?

It performs the early initialization required before your application reaches main().

A simplified conceptual representation:

void Reset_Handler(void)

{

initialize_memory();

initialize_system();

main();

while (1)

{

}

}

The actual startup code is assembly, not simply this C code.

The exact implementation depends on the startup file and toolchain.

6. Why initialize memory?

Your C program may contain:

int counter = 10;

or:

int counter;

The startup code has to prepare the runtime memory environment.

Conceptually:

Flash

│

│ initialized data

▼

SRAM

For example:

int counter = 10;

The initial value 10 needs to be available when the program starts.

Uninitialized global/static variables are normally placed in the .bss section and must be initialized to zero.

So startup code performs tasks such as:

Initialize .data

Clear .bss

Set up runtime environment

↓

main()

This is one reason main() is not the first instruction executed after reset.

7. The complete mental model

Put everything we've learned together:

STM32 RESET

│

▼

Vector Table in Flash

│

┌────────┴────────┐

▼                 ▼

Initial SP        Reset_Handler

│                 │

▼                 ▼

SRAM Stack       Startup Code

│

┌────────┴────────┐

▼                 ▼

.data setup       .bss clear

│                 │

└────────┬────────┘

▼

System initialization

│

▼

main()

│

▼

Your application

The key idea

Reset does not directly jump to main().

It starts through the vector table → Reset_Handler → startup initialization → main().

Quick checkpoint

Answer these 3 without looking back:

Q1. What are the first two important values the Cortex-M obtains from the vector table after reset?

Q2. If the first vector-table entry contains:

0x20005000

what does that value represent?

Q3. Why can't the CPU simply jump directly to main() immediately after reset?