Level 0.3

0.3.1 What is the ALU?

ALU = Arithmetic Logic Unit.

It's the part of the CPU responsible for operations such as:

Arithmetic:

ADD

SUB

...

Logic:

AND

OR

XOR

NOT

Comparisons:

equal?

greater?

less?

So when we conceptually have:

R0 = 10

R1 = 20

R2 = R0 + R1

the flow is roughly:

CPU

│

┌────┴────┐

│ Registers     │

│                      │

│ R0 = 10        │

│ R1 = 20        │

└────┬────┘

│

│

operands

↓

┌─────┐

│ ALU   │

│    +     │

└──┬─┘

│

↓

R2 = 30

Notice something important:

RAM isn't doing the addition.

RAM stores data.

Flash isn't doing the addition.

Flash stores program/data.

The peripheral isn't doing the addition.

The CPU's execution hardware performs it.

But here's where Cortex-M becomes interesting

The CPU needs somewhere very fast to keep values it's actively working with.

That's what the registers are for.

Think of the difference:

CPU

│

┌──────┴──────┐

│ Registers              │

│                               │

│ R0                         │

│ R1                         │

│ R2                        │

│ ...                          │

└──────┬──────┘

│

ALU

│

↓

Result

Registers are directly part of the CPU's programmer-visible state.

RAM is outside the CPU core's register file.

This distinction becomes extremely important when we start pointers and memory access.

🧠 Your first Cortex-M3 register map

For the Cortex-M3, you'll encounter:

R0

R1

R2

R3

R4

R5

R6

R7

R8

R9

R10

R11

R12

SP

LR

PC

xPSR

There are 16 primary 32-bit registers in the Cortex-M programmer's model:

- R0-R12   → General-purpose registers

- R13      → SP (Stack Pointer)

- R14      → LR (Link Register)

- R15      → PC (Program Counter)

And then:

- xPSR     → Program Status Register

Don't try to memorize what every one does yet.

We're going to understand them one by one.

The three registers I want you to remember first

1. PC = Program Counter

The PC tells the processor roughly:

"Where is the next instruction I need to execute?"

Imagine your program contains:

Address       Instruction

0x08000000    MOV R0, #10

0x08000002    MOV R1, #20

0x08000004    ADD R2, R0, R1

0x08000006    ...

Conceptually:

PC

↓

0x08000000

CPU fetches instruction.

Then PC advances.

PC

↓

0x08000002

Fetch next instruction.

And so on.

So the CPU continuously performs something conceptually like:

FETCH

↓

DECODE

↓

EXECUTE

↓

NEXT INSTRUCTION

This is the beginning of understanding how your C code actually becomes a running program.

2. SP = Stack Pointer

SP points to the current location of the stack.

The stack is used for things such as:

- function calls

- local variables

- saved registers

- temporary execution state

We'll spend a LOT more time on this later.

For now:

RAM

┌──────────────────┐

│                                       │

│      Stack                        │ ← SP

│        ↓                           │

│                                       │

│                                       │

└──────────────────┘

This will become very important when we discuss:

void function()

{

int x;

}

and later interrupts and RTOS.

3. LR = Link Register

This one is particularly interesting.

Suppose:

main()

{

function();

}

The CPU needs to remember:

"After function() finishes, where should I return?"

The Link Register helps hold the return address for function calls.

Conceptually:

main()

│

│ call function()

↓

function()

│

│ return

↓

back to main()

The LR helps the processor know where to return.

So now our CPU looks like this

Cortex-M3 CPU

│

┌───────────────┼────────────────┐

│                                  │                                     │

Registers                      ALU                       Control logic

│

┌─────┴───────────────┐

│                                                 │

R0-R12                                 Special

registers

│

┌──────┼──────┐

↓             ↓            ↓

SP             LR              PC

We're slowly building the machine.

🎯 Checkpoint

I don't want to overload you with everything at once.

Answer these without looking back:

Q1

If:

R0 = 15

R1 = 25

and the CPU executes:

R2 = R0 + R1

What component performs the addition?

Q2

What is the purpose of a CPU register?

Q3

What does PC represent?

Q4

What does SP represent?

Q5

Why do we need LR when calling a function?

Q6 ⭐

Imagine:

RAM

┌─────────────┐

│                             │

│                             │

│   Stack                 │ ← SP

│                             │

└─────────────┘

Would you expect the stack to be in RAM or inside the CPU registers?

And why?

My answers:

- alu

- cpu registers are the closest to cpu so that it can accesses data vey fast

- pc represents the next instruction address which cpu needs to execute

- sp represents the next empty location of stack

- we need lr to return to the main program after we calls function

- i expect the stack to be in ram the cpu feteches data from regesters it cannot stores functions and local variables so the stack to be in ram

Refined Correct answers:

Excellent. All six are essentially correct. More importantly, your reasoning is moving in the right direction.

There are two terminology refinements I want you to make:

- Q3: PC is the program counter. In our simplified model, it points to the address of the instruction being executed/fetched next. The exact pipeline behavior is more nuanced, but your model is correct for now.

- Q4: Your answer is correct for our current level. More precisely, SP holds the address of the top/current stack location, not necessarily always "the next empty location." We'll refine stack behavior when we study it properly.

And your Q6 reasoning is good:

The stack is in RAM because local variables and saved execution state need writable runtime memory.

One small correction: the CPU can store some values in registers, and compilers often do exactly that. The stack is needed because runtime data can exceed the available registers and because function calls require persistent execution state.

0.3.2 How does the CPU actually run your program?

Imagine you write:

int a = 10;

int b = 20;

int c;

c = a + b;

You might think:

C code

↓

CPU somehow executes it

But there are several layers between your source code and the CPU.

The real journey is roughly:

YOUR CODE

│

↓

C code

│

↓

Compiler

│

↓

Machine code

│

↓

FLASH

│

↓

CPU

│

┌──────┴──────┐

↓                             ↓

Registers                  ALU

│                             │

└──────┬──────┘

↓

RAM

This is the bridge between Embedded C and hardware.

Step 1: Your C program lives as instructions

Suppose your source code contains:

c = a + b;

The CPU does not understand C.

It understands machine instructions.

The compiler converts your C program into instructions understood by the Cortex-M3 instruction set.

Very simplified:

C:

c = a + b;

↓ compiler

LOAD something

LOAD something

ADD

STORE something

The actual instructions are more specific, but don't worry about assembly syntax yet.

Step 2: Where are those instructions stored?

This is where Flash comes in.

Your STM32F103 has Flash memory.

Conceptually:

FLASH

┌────────────────────────────┐

│ Program instructions                      │

│                                                             │

│ instruction                                         │

│ instruction                                        │

│ instruction                                        │

│ instruction                                         │

└────────────────────────────┘

When you program the STM32, you're essentially putting your compiled program into non-volatile memory.

Then when the MCU powers up:

Flash

↓

CPU fetches instructions

↓

CPU executes them

Step 3: What does PC do?

This is where the Program Counter becomes useful.

Imagine Flash contains:

Address       Instruction

0x08000000    instruction A

0x08000002    instruction B

0x08000004    instruction C

0x08000006    instruction D

The PC keeps track of program execution.

Conceptually:

PC = 0x08000000

↓

fetch A

PC = 0x08000002

↓

fetch B

PC = 0x08000004

↓

fetch C

So the CPU is continuously moving through the instruction stream, except when branches, function calls, interrupts, etc. change the flow.

Step 4: Where does a, b, and c live?

Now we reach RAM.

Your variables need storage.

For example:

RAM

┌──────────────────────┐

│ a = 10                                    │

│ b = 20                                    │

│ c = ?                                       │

│                                                │

│ stack                                      │

│ other runtime data             │

└──────────────────────┘

But here's an important twist:

The compiler doesn't necessarily put every variable in RAM.

It may use registers:

RAM                 CPU

a = 10      →       R0 = 10

b = 20      →       R1 = 20

ALU

↓

R2 = 30

Then the result might eventually be stored back into RAM.

So:

RAM

↓

LOAD

↓

REGISTER

↓

ALU

↓

REGISTER

↓

STORE

↓

RAM

This is a fundamental CPU operation pattern.

Now comes the REALLY important concept

You previously answered:

0x40000000 may be the address of a GPIO pin.

We're going to correct that mental model now.

An address is simply a number identifying a location in an address space.

For example:

0x08000000

could represent a location in Flash.

Another region:

0x20000000

could represent RAM.

And another region:

0x40000000

could belong to the peripheral address space.

So conceptually:

MCU ADDRESS SPACE

0x00000000 ─────────────

Flash

Code

Data

0x1FFFFFFF ─────────────

...

0x20000000 ─────────────

SRAM

Variables

Stack

0x3FFFFFFF ─────────────

...

0x40000000 ─────────────

Peripherals

GPIO

Timers

UART

ADC

...

The exact STM32F103 memory map has specific ranges and details we'll study from the reference manual later.

But the concept is what matters right now.

🔥 Memory-mapped I/O

This is the concept that will eventually make STM32 register programming click.

Imagine a GPIO register has some address:

0x4001080C

Then conceptually:

CPU

│

│ write value

↓

0x4001080C

│

↓

GPIO register

│

↓

GPIO hardware

│

↓

physical pin

So when C code eventually does something like:

*address = value;

the CPU isn't "calling a GPIO function."

It is performing a memory write to a specific address.

Hardware recognizes that address as belonging to a peripheral register.

That's the magic of memory-mapped peripherals.

And this is exactly where your weak area, pointers, becomes extremely relevant.

🧩 Your next challenge

Don't answer immediately from memory. Think about it.

Suppose:

RAM address: 0x20000000

Value:       25

and:

GPIO register address: 0x40000000

Now imagine the CPU executes:

*ptr = 25;

where:

ptr = (uint32_t *)0x40000000;

Q1

What do you think ptr represents?

Q2

Why do we put * before ptr in:

*ptr = 25;

?

Q3

If ptr contains:

0x40000000

does that mean the variable ptr contains the GPIO value, or does it contain something else?

Q4

What's the difference between:

uint32_t x = 25;

and:

uint32_t *ptr = (uint32_t *)0x40000000;

Don't worry about perfect C terminology yet.

Reason it out.

My answers:

- ptr represents gpio register address

- to accesses the address we put * before ptr

- that means ptr contains the address of gpio register so that we can access that gpio and write and read data

- x stores the value 25 in 32 bits and *ptr stores the address 0x40000000

Corrected answers:

Q1. ptr represents GPIO register address

Correct, with one refinement.

uint32_t *ptr = (uint32_t *)0x40000000;

ptr is a pointer variable whose value is:

0x40000000

That value represents the address of whatever memory-mapped register is located there.

So:

ptr

↓

0x40000000

↓

GPIO register

Q2. Why *ptr?

You said:

to access the address we put * before ptr

Almost.

More precisely:

ptr gives us the address. *ptr gives us the value stored at that address.

This distinction is fundamental.

Suppose:

uint32_t x = 25;

Imagine x is located at:

0x20000010

Then:

x

↓

25

&x

↓

0x20000010

And if:

uint32_t *ptr = &x;

then:

ptr

↓

0x20000010

*ptr

↓

25

So:

ptr  = address

*ptr = value at that address

Q3. You got this right

If:

ptr = 0x40000000

then ptr contains the address, not the GPIO data.

So:

ptr

↓

0x40000000

while:

*ptr

↓

value currently stored in the register

For example, if the register contains:

00000001

then:

*ptr == 1

Q4. Here's the correction

You said:

x stores the value 25 in 32 bits and *ptr stores the address 0x40000000.

The first half is correct.

The second half is reversed.

Given:

uint32_t x = 25;

uint32_t *ptr = (uint32_t *)0x40000000;

we have:

x

│

└── stores 25

ptr

│

└── stores 0x40000000

*ptr

│

└── accesses the VALUE at 0x40000000

Therefore:

Address              Value

x            0x20000010            25

ptr          somewhere in RAM      0x40000000

*ptr         accesses              value at

0x40000000

This is the key:

ptr stores an address. *ptr accesses the data located at that address

One analogy

Think of a house.

House address = 25 Main Street

Person inside = Ganesh

Then:

address → tells you WHERE

*address → gives you WHAT IS THERE

For a microcontroller:

0x40000000

↓

WHERE?

↓

GPIO register

*ptr

↓

WHAT'S THERE?

↓

register value

Obviously the CPU isn't literally using houses, but the analogy is useful for the mental model.

Now let's make it more interesting

Suppose:

uint32_t *ptr = (uint32_t *)0x40000000;

and currently:

address 0x40000000 contains:

00000000

Then we execute:

*ptr = 1;

What happens?

We are saying:

Go to address 0x40000000 and write the value 1 there.

Conceptually:

BEFORE

0x40000000

│

↓

00000000

*ptr = 1

AFTER

0x40000000

│

↓

00000001

If that address corresponds to a GPIO output register, hardware can react to that write.

And now we've arrived at the bridge:

C POINTER

↓

MEMORY ADDRESS

↓

PERIPHERAL REGISTER

↓

HARDWARE

↓

PHYSICAL PIN

This is one of the most important mental models in embedded programming.

One last test before we formally move on

Given:

uint32_t *gpio = (uint32_t *)0x40000000;

Answer these:

A

What is stored inside gpio?

B

What does *gpio mean?

C

What does this do?

*gpio = 0x05;

D

What would this do?

uint32_t value = *gpio;

E

What's the difference between:

gpio

and

*gpio

My answers:

- inside gpio the address of some register is stored

- *gpio means it stores the value at that address

- that means at the address of gpio variable the value is write to 5

- it stores 5 in value

- gpio means it contains address and *gpio means it stores the value inside the address

Corrected answers:

1. gpio

You said:

inside gpio the address of some register is stored

✅ Correct.

Given:

uint32_t *gpio = (uint32_t *)0x40000000;

we have:

gpio

│

└──── stores → 0x40000000

That is an address.

2. *gpio

You said:

*gpio means it stores the value at that address

Almost. Say:

*gpio accesses the value at the address stored in gpio.

This distinction becomes important because *gpio itself isn't a separate variable storing something.

Think:

gpio

↓

0x40000000

↓

memory/register at that address

↓

*gpio accesses its value

3. *gpio = 0x05

You said:

at the address of gpio variable the value is write to 5

❌ This is the important correction.

We're not writing to the address of the gpio variable.

We're writing to the address contained inside gpio.

Given:

uint32_t *gpio = (uint32_t *)0x40000000;

*gpio = 0x05;

the operation means:

gpio

│

↓

0x40000000

│

↓

┌──────────────┐

│   register              │

│                              │

│     0x00                 │

└──────────────┘

↓

*gpio = 0x05

↓

┌──────────────┐

│   register              │

│                              │

│     0x05                 │

└──────────────┘

So:

Write 0x05 to address 0x40000000.

That's the exact mental model I want.

4. uint32_t value = *gpio;

You said:

it stores 5 in value

✅ Correct, assuming the register currently contains 0x05.

For example:

register at 0x40000000 = 0x05

Then:

uint32_t value = *gpio;

results in:

value = 0x05

The flow is:

0x40000000

↓

register

↓

0x05

↓

*gpio

↓

value

5. gpio vs *gpio

You said:

gpio means it contains address and *gpio means it stores the value inside the address

You're conceptually correct.

Let's make the professional wording:

gpio

↓

the address

*gpio

↓

the value located at that address

Or even shorter:

gpio = WHERE
*gpio = WHAT IS THERE

Don't jump to GPIO yet

We still have one major piece to understand:

The Cortex-M memory map

We need to answer:

How can Flash, RAM, GPIO, UART, timers, ADC, etc. all have addresses?

Imagine the MCU has one giant address space:

The CPU uses addresses to interact with all of them.

And this is where the concepts you've just learned about pointers finally connect to actual STM32 hardware.

Next lesson: Level 0.4

We'll study:

Memory, addresses and memory-mapped I/O

You'll learn:

- What an address actually represents

- Why RAM has addresses

- Why Flash has addresses

- Why GPIO registers have addresses

- How the CPU distinguishes RAM from peripherals

- STM32F103's actual memory map

- Peripheral address ranges

- Register addresses

- Why volatile is required

- How C pointers interact with hardware

And we'll do actual calculations such as:

Base address + register offset = register address

That last concept is going to make expressions like:

GPIOA->ODR

finally make sense instead of looking like STM32 wizardry.

Level 0.3 pointer checkpoint complete. Next: Level 0.4.