0.7 ARM Cortex-M Programmer's Model

0.7.1 First, what does "programmer's model" mean?

It simply means:

What parts of the CPU are directly visible and usable when we write or debug software?

For Cortex-M3, the core register set is:

R0   R1   R2   R3

R4   R5   R6   R7

R8   R9   R10  R11

R12

R13 = SP

R14 = LR

R15 = PC

xPSR

The registers are 32-bit on Cortex-M3.

A useful picture:

Cortex-M3 CPU

│

┌────────────┴────────────┐

│                                                          │

General registers                   Special registers

R0-R12                                   SP  LR  PC  xPSR

│                                                          │

└──────────┬──────────────┘

│

ALU

0.7.2 R0-R12: General-purpose registers

These are the CPU's main working registers.

Suppose conceptually:

R0 = 10

R1 = 20

The ALU can perform:

R0 + R1

and produce:

R2 = 30

So:

R0 ──┐

├──> ALU ──> R2

R1 ──┘

This connects directly to what you already learned about the ALU and registers. Your previous material specifically showed that registers hold values the CPU is actively working with, while RAM is external storage for runtime data.

Important

Don't think:

"R0 is permanently for variable a."

No.

The compiler decides how registers are used.

For now:

R0-R12 = general CPU working registers.

0.7.3 R13 = SP

This is where things get interesting.

R13 = SP

SP means:

Stack Pointer

You've already learned that the stack exists in SRAM.

The SP contains the address associated with the current stack position.

For example:

SP = 0x20001000

means the SP contains an address in SRAM.

It does not mean:

SP contains the stack itself

Think:

CPU

│

│ SP = 0x20001000

│

▼

SRAM

0x20001000

│

▼

current stack position

Your earlier checkpoint specifically refined this distinction: SP holds the current stack position/address, rather than necessarily meaning "the next empty location."

0.7.4 What is the stack actually for?

We'll study the stack properly later, but for now remember that it is used for things such as:

- function calls

- local variables

- saved registers

- temporary execution state

For example:

void function(void)

{

int x;

}

The runtime needs somewhere to keep information associated with function().

That's where the stack becomes important.

SRAM

┌──────────────────┐

│                                        │

│      Stack                        │

│        ▲                           │

│        │                             │

│        SP                          │

│                                       │

│                                        │

└──────────────────┘

We will return to this when we study startup, interrupts, and eventually RTOS.

0.7.5 R14 = LR

R14 = LR

LR means:

Link Register

Its major role is associated with function calls and returning from them.

Imagine:

int main(void)

{

calculate();

}

Execution conceptually becomes:

main()

│

│ call

▼

calculate()

│

│ return

▼

main()

The processor needs to preserve information about where it should return.

That's where LR comes into play.

main()

│

│

└──► calculate()

│

│

└──► LR contains return information

Your existing Level 0.3 material introduced exactly this role for LR.

We'll see the actual ARM instructions later, so don't worry about assembly yet.

0.7.6 R15 = PC

R15 = PC

PC means:

Program Counter

This is probably the most intuitive special register.

Your program consists of instructions stored in memory.

Conceptually:

Address       Instruction

────────────────────────────

0x08000000    Instruction A

0x08000002    Instruction B

0x08000004    Instruction C

0x08000006    Instruction D

The processor needs to keep track of its instruction flow.

That's the job of the PC.

PC

│

▼

Fetch instruction

│

▼

Decode

│

▼

Execute

│

▼

Next instruction

│

└──────────► PC

Your earlier material used this exact fetch → decode → execute model.

Small precision point

At our level, saying:

"PC points to the next instruction"

is perfectly useful.

Technically, Cortex-M has instruction pipelining, so the exact observable PC behavior can be more nuanced.

We'll deal with that later when we reach assembly.

For now:

PC = controls instruction flow.

0.7.7 xPSR

Now we have:

xPSR

This is the:

Program Status Register

It contains processor execution/status information.

One important category is the condition flags produced by operations:

N = Negative

Z = Zero

C = Carry

V = Overflow

For example:

5 - 5 = 0

The result is zero, so the processor can have:

Z = 1

A later conditional instruction can use that status.

Conceptually:

ALU

│

│ result

▼

xPSR flags

│

├── N

├── Z

├── C

└── V

We won't dive into every xPSR field yet. That's something we'll connect later to assembly and conditional execution.

0.7.8 The three most important special registers

If you forget everything else today, remember this:

R13 → SP → Stack

R14 → LR → Function return

R15 → PC → Instruction flow

That's worth memorizing.

0.7.9 MSP and PSP

Now the Cortex-M-specific twist.

There are two stack pointers:

MSP

PSP

MSP = Main Stack Pointer

This is the main stack pointer used by the processor.

After reset, the processor starts using the MSP.

It is also important when dealing with exceptions and interrupts.

PSP = Process Stack Pointer

This provides another stack pointer that can be used for process/application execution.

This becomes very important later when we study RTOS.

For example, an RTOS may have multiple tasks:

Task A → its stack

Task B → its stack

Task C → its stack

The Cortex-M stack-pointer architecture gives us mechanisms that make task/context management possible.

For now, keep it simple:

Cortex-M

│

┌───────┴───────┐

│                                   │

MSP                            PSP

│                                   │

Main/system      Process/task

stack                           stack

0.7.10 The complete picture

Now look at the whole programmer's model:

Cortex-M3

│

┌─────────────────┴─────────────────┐

│                                                                                │

R0-R12                                                           Special registers

│                                                                               │

General-purpose                                                   ┌──┼─────────────┐

working values                                                     │     │               │             │

SP     LR            PC          xPSR

│      │               │             │

Stack Return     Flow     Status

And the numbering:

R0-R12 → General purpose

R13 → SP

R14 → LR

R15 → PC

That relationship is fundamental.

0.7.11 Connect this to what we've already learned

This is the really important part.

You've already learned:

GPIOA_ODR |= (1 << 5);

At the C level, you understand:

GPIOA_ODR

↓

memory-mapped register

↓

set bit 5

↓

GPIO hardware

Now we're adding the CPU side:

C statement

↓

Compiler

↓

Machine instructions

↓

CPU

┌───┼──────────────┐

│       │                                 │

PC   Registers                  ALU

│       │                                │

│   └──────┐                    │

│                  └─----──────┘

│

▼

Bus

│

▼

GPIO register

│

▼

PA5

This is the bridge we're going to complete in 0.12.

Your 0.7 mental cheat sheet

| Register | Meaning | Main idea |
| --- | --- | --- |
| R0-R12 | General-purpose | CPU working values |
| R13 / SP | Stack Pointer | Current stack position |
| R14 / LR | Link Register | Function-return information |
| R15 / PC | Program Counter | Instruction flow |
| xPSR | Program Status Register | Processor status/flags |
| MSP | Main Stack Pointer | Main/system stack |
| PSP | Process Stack Pointer | Process/task stack |

One important distinction

Don't confuse:

SP

with:

MSP / PSP

Think of SP as the currently active stack-pointer register, while MSP and PSP are the two stack-pointer mechanisms available in Cortex-M. The processor's current mode determines which one is used.

Quick checkpoint

Just 3 questions, then we'll continue.

Q1

If:

R13 = 0x20001000

what does this tell you?

Q2

If the CPU calls:

function();

which register is associated with the return information?

Q3

Put these in the correct relationship:

R15

R14

R13

PC

LR

SP

Answers:

- You said:

"it is the address of the register"

Not quite.

If:

R13 = 0x20001000

then:

R13 = SP

and 0x20001000 is the address of the current stack position.

It is not the address of the SP register itself.

Think:

CPU

│

R13 / SP

│

│ contains

▼

0x20001000

│

▼

SRAM

│

▼

current stack position

This distinction is important because later we'll be dealing with pointers and addresses constantly.

2.  LR

3.  13 - SP, 14 - LR, 15 – PC

Let's finish 0.7

We have the basic register map. Now let's understand the two stack pointers properly.

MSP vs PSP

Cortex-M provides two stack pointers:

MSP = Main Stack Pointer

PSP = Process Stack Pointer

The important relationship is:

SP

│

┌──────┴──────┐

│                               │

MSP                        PSP

But SP is not a third physical stack pointer.

The processor has two stack-pointer registers, MSP and PSP, and the currently selected one is what we refer to through the active SP.

After reset

The processor starts using:

MSP

This is why startup code and the initial stack setup are closely connected. We'll explore that in 0.11 Startup Sequence.

Why PSP?

PSP becomes useful when the processor is running application/process code and especially when we later study an RTOS.

For example:

RTOS

│

┌──────┼──────┐

│              │              │

Task A   Task B    Task C

│              │             │

Stack     Stack      Stack

The idea of separate process/task stacks is one reason PSP becomes important.

You don't need to memorize the detailed CONTROL-register rules yet. We'll introduce those when they become relevant.

xPSR, one step deeper

We know:

xPSR = Program Status Register

For now, remember three groups inside it:

xPSR

│

├── APSR → arithmetic/status flags

├── IPSR → exception/interrupt state

└── EPSR → execution state

You don't need to memorize their fields yet.

The useful mental model is:

CPU performs operation

│

▼

status changes

│

▼

xPSR

For example, arithmetic can affect:

N → negative

Z → zero

C → carry

V → overflow

Later, when we study interrupts, IPSR will become much more meaningful because it tells us about the current exception/interrupt context.

0.7 is now almost complete

Our mental model is:

Cortex-M3

│

├── R0-R12

│    └── General-purpose working registers

│

├── R13 / SP

│    └── Active stack pointer

│

├── R14 / LR

│    └── Function return information

│

├── R15 / PC

│    └── Instruction flow

│

└── xPSR

└── Processor status/execution information

Stack pointers:

├── MSP

└── PSP

One final checkpoint for 0.7

Answer these 3, then I'll mark 0.7 complete:

Q1. After a Cortex-M3 reset, which stack pointer is initially used?

Q2. What is the basic difference between MSP and PSP?

Q3. What kind of information does xPSR contain?

Answers:

- MSP

- msp is used when the mcu is reset and psp is used in rtos like multiple tasks so that we can access separate task

- status of the results when the program executed

Actual Answers:

Q1. Which stack pointer is initially used after reset?

Your answer: MSP ✅

Correct.

After reset, the Cortex-M starts execution using the Main Stack Pointer (MSP).

Q2. MSP vs PSP

Your answer is correct ✅.

A slightly more precise version:

MSP is the main stack pointer and is used by default after reset and for exception/interrupt handling. PSP can be used for application/process execution and is particularly useful in RTOS environments where different tasks have their own stacks.

Your RTOS connection is exactly the important reason we are learning this now.

Cortex-M

│

┌────────┴────────┐

│                                       │

MSP                               PSP

│                                       │

System/exception    Application/

stack                           task stack

Q3. What does xPSR contain?

Your answer:

"status of the results when the program executed"

Correct idea ✅, but let's make it more precise.

xPSR contains information about the processor's current execution and status state, including condition flags such as N, Z, C, and V.

For example:

5 - 5 = 0

↓

Z flag

↓

Z = 1

So don't think of xPSR as simply "the result." It stores status information about execution and operations.

0.7 COMPLETE

You've now covered:

0.7 ARM Cortex-M Programmer's Model

R0-R12  → General-purpose registers

R13     → SP

R14     → LR

R15     → PC

xPSR    → Processor status/execution state

MSP     → Main Stack Pointer

PSP     → Process Stack Pointer

Our Level 0 progress is now:

0.1 MCU fundamentals          ✅

0.2 ARM ecosystem             ✅

0.3 STM32 ecosystem           ✅

0.4 Cortex-M3 architecture    ✅

0.5 Memory                    ✅

0.6 Registers & bit manipulation ✅

0.7 Programmer's model        ✅

0.8 Buses                     ← NEXT

0.9 Clock & reset

0.10 Interrupt fundamentals

0.11 Startup sequence

0.12 C → Assembly → Hardware