LEVEL 0.4

Memory, Addresses & Memory-Mapped I/O

Our goal is to understand this:

GPIOA->ODR |= (1 << 5);

without treating it as magic.

Eventually, you'll understand it through a chain like:

C code

↓

pointer

↓

address

↓

bus

↓

peripheral register

↓

GPIO hardware

↓

physical pin

Let's build that chain from the beginning.

0.4.1 What is memory?

At the simplest level, memory is a collection of locations capable of storing information.

Imagine RAM like a huge row of boxes:

RAM

Address          Data

────────────────────────

0x20000000   →   25

0x20000004   →   100

0x20000008   →   0

0x2000000C   →   55

0x20000010   →   ...

Each location has an address.

The CPU can say:

"Give me the data stored at this address."

or:

"Store this data at this address."

For example:

READ:

CPU ──────→ address 0x20000000

CPU ←────── value 25

Or:

WRITE:

CPU ──────→ address 0x20000000

CPU ──────→ value 50

Afterwards:

0x20000000 → 50

That's the fundamental idea of memory access.

0.4.2 Why do addresses exist?

Imagine you have 1 MB of RAM.

The CPU needs some way to identify which byte it wants.

That's what addresses provide.

Think:

Address = location

Data    = information stored there

For example:

Address              Data

0x20000000             25

0x20000001             10

0x20000002             7

0x20000003             99

The addresses identify locations.

The values are the contents.

0.4.3 Address ≠ data

This is worth drilling into because you had a slight confusion about this earlier.

Suppose:

0x20000000 → 25

Then:

0x20000000

is the address.

And:

25

is the data stored at that address.

Never mix these two.

WHERE?

↓

ADDRESS

WHAT?

↓

DATA

This is exactly why pointers exist.

0.4.4 Connecting this to C

Consider:

uint32_t x = 25;

Conceptually, suppose the compiler places x at:

0x20000000

Then:

x

↓

25

while:

&x

means:

"Give me the address of x."

So:

&x

↓

0x20000000

And:

*x

would not be correct because x isn't a pointer.

Instead:

uint32_t *ptr = &x;

Now:

ptr

↓

0x20000000

*ptr

↓

25

You already learned this in the previous checkpoint.

Now we're going to apply exactly the same concept to hardware.

0.4.5 Here's the STM32 trick

The CPU doesn't only use addresses for RAM.

It can also use addresses for peripherals.

This is called:

Memory-Mapped I/O

Instead of having a completely separate mechanism like:

CPU → special GPIO command

the MCU maps peripheral registers into the processor's address space.

So conceptually:

CPU ADDRESS SPACE

│

┌───────────┼─────────────┐

│                         │                              │

↓                      ↓                            ↓

FLASH              RAM              PERIPHERALS

│

┌─────────┼─────────┐

↓                   ↓                   ↓

GPIO            UART            TIMER

The CPU can access a peripheral register using an address.

0.4.6 Imagine a GPIO register

Let's make a fictional example.

Suppose:

GPIO register address = 0x40000000

And the register currently contains:

00000000

We create:

uint32_t *gpio = (uint32_t *)0x40000000;

Now:

gpio

↓

0x40000000

If we execute:

*gpio = 1;

we're telling the CPU:

Write the value 1 to address 0x40000000.

Hardware sees that address and recognizes:

"That's one of my peripheral registers."

The GPIO hardware responds to the write.

Conceptually:

CPU

│

│ WRITE

│ address = 0x40000000

│ value   = 1

↓

BUS

│

↓

GPIO peripheral

│

↓

GPIO register

│

↓

GPIO hardware

│

↓

Pin changes state

This is the foundation of register-level programming.

0.4.7 But how does the CPU know where GPIO is?

This is where the memory map comes in.

The MCU designer defines regions of the address space.

For example, conceptually:

Address Space

──────────────────────────────

0x00000000

│

│

↓

FLASH

│

│

↓

...

0x20000000

│

│

↓

SRAM

│

│

↓

...

0x40000000

│

│

↓

PERIPHERALS

│

├── GPIO

├── UART

├── SPI

├── I²C

├── TIMER

└── ADC

The exact STM32F103 ranges and addresses we'll take directly from the reference manual when we get to the device-specific portion.

For now, understand the architecture:

Different hardware resources occupy different regions of the MCU's address space.

0.4.8 Address vs register

This distinction is crucial.

Suppose:

GPIOA base address = 0x40010800

That's the base address of the GPIOA peripheral/register block, not necessarily one particular register.

Inside GPIOA there are multiple registers.

Conceptually:

GPIOA

Base = 0x40010800

+0x00 → Register A

+0x04 → Register B

+0x08 → Register C

+0x0C → Register D

...

Therefore:

Base address

+

Register offset

=

Register address

For example, if:

Base = 0x40010800

Offset = 0x0C

then:

0x40010800

+       0x0C

─────────────

0x4001080C

That is the basic mechanism used to organize peripheral registers.

0.4.9 Why are there offsets?

Because a peripheral isn't just one register.

GPIO needs multiple pieces of configuration and status.

Conceptually:

GPIOA

│

├── Configuration register

├── Input data register

├── Output data register

├── Set/reset register

└── ...

Each gets an address.

So:

GPIOA base

│

├── + offset → configuration

├── + offset → input

├── + offset → output

└── + offset → set/reset

This will become extremely important when we eventually open the STM32F103 reference manual.

0.4.10 Now let's connect everything

Suppose:

GPIOA base = 0x40010800

ODR offset = 0x0C

Then:

GPIOA ODR address

=

0x40010800 + 0x0C

=

0x4001080C

Imagine bit 5 of ODR controls our output pin.

Then:

*(volatile uint32_t *)0x4001080C |= (1 << 5);

Let's dissect this monster:

0x4001080C

↓

GPIOA ODR address

↓

cast as pointer

↓

dereference

↓

access GPIOA ODR

↓

OR with (1 << 5)

↓

set bit 5

↓

GPIO hardware reacts

↓

PA5 changes state

This is the destination we're working toward.

Don't worry if the complete expression still looks ugly. We'll break down volatile, casting, bit shifting, and registers separately.

0.4.11 One critical concept: volatile

You're going to see this everywhere in embedded C:

volatile uint32_t *

Why?

Because a hardware register can change without the compiler's normal understanding of program flow.

For example:

while(1)

{

if(*status_register == 1)

{

...

}

}

The hardware might change status_register.

If the compiler incorrectly assumes:

"This memory can't magically change, so I'll just reuse the previous value."

we have a disaster.

volatile tells the compiler:

"Treat accesses to this object as observable. Actually perform the memory access."

We'll study exactly what that means later.

For now:

volatile

↓

"Hardware may change this"

That's the intuition.

Important distinction

There are three things here:

ADDRESS

↓

LOCATION

REGISTER

↓

HARDWARE STORAGE/CONTROL FIELD AT THAT LOCATION

VALUE

↓

BITS CURRENTLY STORED IN THE REGISTER

For example:

0x4001080C

↓

ADDRESS

↓

GPIO ODR

↓

00000000

↓

REGISTER VALUE

If we set bit 5:

00000000

↑

bit 5

becomes

00100000

The GPIO hardware sees the changed register state and changes its output accordingly.

🎯 Your checkpoint

This is an important one. Take your time.

Q1

Suppose:

Address: 0x20000000

Value:   50

Which is the address and which is the data?

Q2

If:

uint32_t *ptr = (uint32_t *)0x20000000;

what does:

ptr

contain?

And what does:

*ptr

represent?

Q3

Suppose:

Peripheral base address = 0x40010000

Register offset         = 0x20

Calculate the register address.

Q4

Why can a GPIO register have an address just like RAM does?

Q5 ⭐

Explain memory-mapped I/O in your own words.

Try to include:

CPU

↓

address

↓

bus

↓

peripheral register

↓

hardware

Q6

Consider:

uint32_t *gpio = (uint32_t *)0x40000000;

*gpio = 0x05;

Explain exactly what happens conceptually, step by step.

Don't worry about volatile yet.

My answers:

- 0x20000000 is address and 50 is data

- ptr contains the address 0x20000000 and *ptr contains the data present at that address

- then the output data register address is 0x40010000 + 0x20

- every data storing variables and peripherirals need to be stored somewhere in memory of mcu so to access that values we address every variable with address

- ---------

- if the value at address 0x40000000 is 00000000 then it will become 00000005

Corrected answers:

Q1

0x20000000 = address

50          = data

Exactly.

Q2

Correct:

ptr

↓

0x20000000

*ptr

↓

data at 0x20000000

Q3

You correctly identified the operation, but you didn't finish the calculation.

0x40010000

+      0x20

────────────

0x40010020

So the register address is:

0x40010020

Also, I deliberately said "register offset", not "ODR offset." We shouldn't assume which register it is until the MCU documentation tells us.

Q4

Your reasoning is close, but there's an important distinction.

Peripherals aren't necessarily "stored in memory" in the same way that variables are.

Instead, the MCU's address space maps certain addresses to peripheral registers.

Think:

Address space

│

├── Flash addresses

├── SRAM addresses

└── Peripheral addresses

│

├── GPIO registers

├── UART registers

├── Timer registers

└── ADC registers

So when the CPU accesses a peripheral address, the bus/interconnect routes that access to the corresponding peripheral hardware.

This is memory-mapped I/O.

Q5 ❌ / incomplete

You left this blank, and that's okay. This is the concept I specifically want you to master.

A good answer would eventually be:

Memory-mapped I/O is a technique where peripheral registers are assigned addresses in the MCU's address space, allowing the CPU to interact with hardware by performing normal memory read/write operations.

We'll make this intuitive rather than memorized.

Q6 🟡

You correctly understood the final result:

Before:

0x40000000 → 00000000

*gpio = 0x05

After:

0x40000000 → 00000101

But there's one important missing piece:

We are not writing to the address of the gpio variable.

We're writing to the address stored inside gpio.

gpio

│

└──────→ 0x40000000

│

↓

[register]

│

↓

0x05

That's the pointer concept I want burned into your brain before we proceed.

Let's finish 0.4 properly

Before we look at the real STM32F103 memory map, I want one final conceptual test.

Suppose we have:

GPIO register address = 0x40000000

Current register value = 0xA0

And:

uint32_t *gpio = (uint32_t *)0x40000000;

Question 1

What is:

gpio

?

Question 2

What is:

*gpio

?

Question 3

What happens after:

*gpio = 0x05;

?

Question 4

What happens after:

*gpio |= 0x05;

starting from the original value 0xA0?

Write the values in binary if you can.

Question 5 ⭐

In your own words, explain:

What is memory-mapped I/O?

Don't copy my definition. Explain it the way you understand it.

My answers:

- gpio is the address storing variable

- *gpio stores the value present at that address

- it writes the value in gpio address to 00000005

- it set the 5th bit in address

- the mcu assigns some address values in order to access them easily

Corrected answers:

1. gpio

You said:

gpio is the address storing variable

Almost.

Given:

uint32_t *gpio = (uint32_t *)0x40000000;

gpio is a pointer variable.

Its value is:

gpio

↓

0x40000000

So:

gpio is a variable that stores an address.

Not "the address storing variable."

2. *gpio

You said:

*gpio stores the value present at that address

Very close, but use accesses, not stores.

gpio

↓

0x40000000

↓

register

↓

0xA0

Therefore:

gpio  → address

*gpio → value at that address

If we do:

uint32_t x = *gpio;

we read the register.

If we do:

*gpio = 0x05;

we write to the register.

3. *gpio = 0x05

You said:

it writes the value in gpio address to 00000005

The idea is correct.

More precisely:

It writes 0x05 to the memory/register location whose address is stored in gpio.

Since:

gpio = 0x40000000

we get:

*gpio = 0x05

meaning:

address 0x40000000

↓

00000101

4. *gpio |= 0x05

You said:

it set the 5th bit in address

❌ This is the important mistake.

It does not modify the address.

It modifies the value stored at that address.

Let's see it.

Original value:

0xA0

Binary:

1010 0000

And:

0x05

is:

0000 0101

Now:

1010 0000

| 0000 0101

------------

1010 0101

Therefore:

Before:

0xA0

After:

0xA5

The address remains:

0x40000000

Only the data/register value changes.

This distinction is extremely important:

5. Memory-mapped I/O

You said:

MCU assigns some address values in order to access them easily.

You're on the right track, but we need to make it more precise.

The better mental model is:

The MCU's address space assigns specific address ranges to hardware resources such as Flash, SRAM, and peripheral registers. The CPU can then access those peripheral registers using normal memory read/write operations.

So:

That is memory-mapped I/O.

The distinction I want you to master

Let's put everything together.

Suppose:

uint32_t *gpio = (uint32_t *)0x40000000;

Then:

Therefore:

gpio

↓

ADDRESS

while:

*gpio

↓

VALUE AT THAT ADDRESS

And:

*gpio = 0x05;

means:

WRITE VALUE

│

↓

0x05

│

↓

ADDRESS STORED IN gpio

│

↓

0x40000000

one last exercise

Given:

uint32_t *REG = (uint32_t *)0x40001000;

and initially:

Address 0x40001000 → 0x80

Answer:

A.

What is the value of REG?

B.

What is the value of *REG?

C.

After:

*REG = 0x12;

what is at address 0x40001000?

D.

After that, execute:

*REG |= (1 << 3);

What is the new value?

E.

Does the address 0x40001000 change at any point? Why?

My answers:

A. 0x40001000

B. 0x80

C. i think 0x80+0x12

D. the 3rd bit form right side will be set

E. no the adress do not change, when the value changes then it may be stored in some other location

Corrected answers:

A. What is REG?

You answered:

0x40001000

✅ Correct.

uint32_t *REG = (uint32_t *)0x40001000;

means:

REG → 0x40001000

REG is a pointer whose value is an address.

B. What is *REG?

You answered:

0x80

✅ Correct.

Initially:

0x40001000 → 0x80

Therefore:

REG  = 0x40001000

*REG = 0x80

Perfect.

C. *REG = 0x12

You answered:

0x80 + 0x12

❌ This is the important correction.

An assignment does not add the new value to the old value.

*REG = 0x12;

means:

Replace the value at the address with 0x12.

Before:

Address       Value

0x40001000 →  0x80

After:

Address       Value

0x40001000 →  0x12

The 0x80 is gone.

If we wanted addition, we'd write something like:

*REG = *REG + 0x12;

which would give:

0x80 + 0x12 = 0x92

But:

*REG = 0x12;

gives:

0x12

Remember:

=    → replace

+=   → add

|=   → OR / set selected bits

&=   → AND / clear selected bits depending on mask

^=   → XOR / toggle selected bits

These operators are going to become your daily vocabulary in register programming.

D. *REG |= (1 << 3)

You said:

the 3rd bit from right side will be set

🟡 Almost.

After Q3, the register contains:

0x12

Convert it:

0x12 = 0001 0010

Now:

(1 << 3)

means:

0000 0001

↓ shift left 3

0000 1000

So:

0001 0010

| 0000 1000

------------

0001 1010

Therefore:

New value = 0x1A

And technically we're setting bit 3, which is the fourth bit from the right if you're counting positions naturally from 1.

The numbering is:

Bit:     7 6 5 4 3 2 1 0

↓ ↓ ↓ ↓ ↓ ↓ ↓ ↓

0 0 0 0 0 0 0 0

↑

bit 3

This is why embedded engineers say "set bit 3", not "set the third bit."

E. Does the address change?

You answered:

no the address do not change, when the value changes then it may be stored in some other location

First part:

✅ The address does not change.

Second part:

❌ The value doesn't move to another location just because it changes.

Think of it like this:

ADDRESS                 DATA

0x40001000  ─────────→  0x80

Then:

*REG = 0x12;

becomes:

ADDRESS                 DATA

0x40001000  ─────────→  0x12

Same location.

Then:

*REG |= (1 << 3);

becomes:

ADDRESS                 DATA

0x40001000  ─────────→  0x1A

Still the same address.

The data changed. The location didn't.

One final question:

Given:

uint32_t *REG = (uint32_t *)0x40001000;

and:

0x40001000 → 0x20

What are the values of:

REG

*REG

Then what happens to the register value after:

*REG = 0x50;

And finally:

*REG |= (1 << 2);

What is the final hexadecimal value?

My Answer:

reg 0x40001000

*reg 0x20

then *reg becomes 0x50

final hex value is 0x54

all are correct.