1.3 STM32 Documentation

This is a short but very important section.

Until now, I've been giving you the register addresses and explaining what they mean. A professional embedded engineer cannot depend on someone handing over every address.

You need to be able to go:

"I need to configure X. Which document tells me how?"

For STM32, there are three documentation layers we care about.

1. Datasheet

The datasheet is primarily about the specific STM32 device.

For our target:

STM32F103C8T6

Use the datasheet for things like:

- Pinout

- Package information

- Memory capacity

- Electrical characteristics

- Operating conditions

- Pin functions / alternate functions

- Device-specific specifications

Think:

DATASHEET

↓

"What does this particular chip provide?"

For example:

Which physical pin corresponds to PA5?

The datasheet is the appropriate place to look.

2. Reference Manual

This is the document you'll use constantly once we reach Level 2 and beyond.

For STM32F1, the relevant reference manual is:

RM0008: STM32F101xx, STM32F102xx, STM32F103xx, STM32F105xx and STM32F107xx advanced ARM-based 32-bit MCUs

Use the reference manual for:

- GPIO operation

- RCC

- Timers

- USART

- SPI

- I²C

- ADC

- Interrupt-related peripheral configuration

- Register descriptions

- Bit fields

- Peripheral behavior

- Clock configuration

Think:

REFERENCE MANUAL

↓

"How does this peripheral actually work?"

For example:

What does bit 2 of RCC_APB2ENR do?

Reference Manual.

3. ARM Cortex-M documentation

The STM32F103 uses a Cortex-M3 core.

So there is another layer of documentation concerning the processor itself.

This is where you look for concepts such as:

- CPU architecture

- Registers

- Exceptions

- NVIC

- instruction behavior

- processor execution

- Cortex-M programmer's model

Think:

ARM DOCUMENTATION

↓

"How does the Cortex-M3 CPU itself work?"

4. The three-document mental model

This is worth memorizing:

Example

Suppose you ask:

"Can PA5 be used as a particular alternate function?"

Start with the datasheet for the device's pin/alternate-function information.

Suppose you ask:

"How do I configure the GPIO peripheral?"

Go to RM0008.

Suppose you ask:

"What exactly does the NVIC do?"

Look at the Cortex-M3 documentation.

5. Another important document: Errata

There is also an errata sheet.

This documents known device limitations or unexpected behaviors.

Think:

Datasheet

+

Reference Manual

+

Errata

For professional work, this matters because:

The reference manual tells you how the device is supposed to behave.
The errata can tell you about known deviations or limitations.

We don't need to study the entire errata sheet now.

Just know that it exists.

6. How we'll use documentation in this course

Here's an important change from our earlier learning.

Suppose Level 2 asks:

Configure PA5 as a 2 MHz general-purpose push-pull output.

I won't simply say:

GPIOA->CRL = ...

and ask you to memorize it.

We'll do:

Requirement

↓

Find GPIO chapter in RM0008

↓

Find GPIOA registers

↓

Find CRL

↓

Find PA5 field

↓

Read MODE/CNF definitions

↓

Construct bit configuration

↓

Write C

↓

Test

That workflow is far more valuable than memorizing register values.

7. Search strategy

When you're inside a large reference manual, don't read it from page 1.

Use the PDF search.

For example:

Need GPIOA clock

↓

Search:

"APB2 peripheral clock enable register"

or:

Need GPIO output configuration

↓

Search:

"GPIO port configuration register low"

Then verify that you've found the correct register, not just a similar-looking occurrence.

8. One professional habit: verify context

Suppose you search for:

RCC_APB2ENR

and find:

Bit 2: IOPAEN

Don't immediately copy it.

Check:

Which device family?

Which peripheral?

Which register?

Which bit?

What does the bit mean?

What is the reset value?

This prevents a very common embedded mistake:

Taking a correct register definition from the wrong STM32 family or peripheral.

That's especially important because STM32 has many families:

F0

F1

F2

F3

F4

G0

G4

H5

H7

...

The concepts transfer, but registers and peripherals are not guaranteed to be identical.

9. Our documentation hierarchy

For this course, we'll primarily use:

And for Cortex-M3-specific architecture:

ARM

↓

Cortex-M3 documentation

↓

CPU architecture

When we need to verify a technical detail, I'll prefer the appropriate primary documentation rather than relying on a random tutorial/blog.

🧪 1.3 Quick checkpoint

No need to search anything yet.

Q1

You want to know:

How many KB of Flash and SRAM does the STM32F103C8T6 have?

Which document should you primarily check?

Q2

You want to know:

Which bit in which register enables the GPIOA peripheral clock?

Which document?

Q3

You want to understand:

What does the Cortex-M3 NVIC do?

Which documentation layer?

Answers:

- datasheet

- reference manual

- ARM Cortex-M documentation

Exactly. 3/3 correct. ✅

Your documentation decision-making is now correct:

| Question | Primary source |
| --- | --- |
| Flash/SRAM capacity, pinout, electrical specs | Datasheet |
| GPIO/RCC/timer/USART registers and behavior | Reference Manual (RM0008) |
| Cortex-M3 CPU, NVIC, exceptions, core architecture | ARM Cortex-M documentation |

One important professional habit to add:

The document is only half the answer. Always verify that the document applies to your exact MCU/family.

For example, don't blindly take an STM32F4 register definition and apply it to the F103 because both say "STM32."

1.3 STM32 Documentation: ✅ COMPLETE

Our current Level 1 progress:

1.1 STM32F103C8T6 Orientation       ✅

1.2 Memory & Device Organization    ✅

1.3 STM32 Documentation             ✅

1.4 STM32CubeIDE Environment        🔜

1.5 Project Anatomy

1.6 First STM32 Program

1.7 Build System

1.8 Flashing & Programming

1.9 Debugging Fundamentals

1.10 Startup & Execution Flow

1.11 CMSIS / HAL / Software Layers

1.12 Clock Configuration Awareness

1.13 Coding Structure

1.14 Hands-on Exercise

1.15 Professional Checkpoint

Next: 1.4 STM32CubeIDE Environment

This is where we switch from mostly theory to hands-on work.

We'll open CubeIDE and learn the environment practically, including:

Workspace → Project → .ioc → Device Configuration → Code Generation → Project Explorer → Build → Debug