Level 0.5: STM32F103C8T6 Memory Map

First, let's establish the real map.

The official datasheet gives the following major address regions:

STM32F103C8T6

32-bit address space

0x0000 0000

│

│  CODE / BOOT MEMORY REGION

│

▼

0x2000 0000

│

│  SRAM

│

▼

0x4000 0000

│

│  PERIPHERALS

│

▼

0xE000 0000

│

│  Cortex-M3 INTERNAL

│  PERIPHERALS

│

▼

But here's where we need to become more precise than our earlier simplified explanation.

1. Code region: 0x00000000

RM0008 says the code area starts at:

0x0000 0000

and is accessed through the Cortex-M3 ICode/DCode buses. The boot memory is mapped into this code area depending on the boot configuration.

For normal execution from internal Flash, the STM32's Flash is available through this code/boot mapping.

The official datasheet's memory map also shows Flash starting at:

0x0800 0000

with the 0x00000000 region involved in the boot-memory aliasing mechanism.

So don't memorize:

0x00000000 = Flash

as an absolute statement.

Instead:

0x00000000 is the Cortex-M code/boot region, where the STM32 can map boot memory.

That's a much more professional understanding.

2. SRAM: 0x20000000

This one is straightforward.

0x2000 0000

│

└── SRAM

RM0008 explicitly states that the SRAM data area starts at 0x20000000.

This is where things such as:

int temperature;

int counter;

char buffer[100];

can live during program execution.

And remember what we learned earlier:

Flash → program/code

SRAM  → runtime data

Stack → SRAM

For the STM32F103C8, the device has 20 KB SRAM according to ST's product information for the family.

3. Peripheral region: 0x40000000

Now things get interesting.

The official STM32F103 memory map places the peripherals beginning at:

0x4000 0000

And this region contains many different peripherals.

For example, the official map includes:

0x4000 0000   TIM2

0x4000 0400   TIM3

0x4000 0800   TIM4

...

0x4000 4400   USART2

0x4000 4800   USART3

...

0x4000 5400   I2C1

...

0x4000 6400   CAN

...

0x4000 8800   GPIOA

0x4000 8C00   GPIOB

0x4001 1000   GPIOC

...

0x4001 2800   ADC1

0x4001 2C00   ADC2

0x4001 3000   TIM1

0x4001 3400   SPI1

0x4001 3800   USART1

...

0x4002 1000   RCC

These addresses come directly from ST's memory map.

Now notice something beautiful:

Our previous hypothetical

GPIO base = 0x40010800

was not random at all.

The official STM32F103 memory map shows:

GPIOA = 0x40010800

GPIOB = 0x40010C00

GPIOC = 0x40011000

GPIOD = 0x40011400

GPIOE = 0x40011C00

So now our earlier pointer exercises are starting to become actual STM32 programming.

4. The crucial architecture

Here's the mental model I want you to develop:

STM32F103

│

▼

CPU

│

Bus system

│

┌────────────┼────────────┐

▼                         ▼                         ▼

Flash                    SRAM             Peripherals

│

┌────────┬───────────┼──────────┐

▼                ▼                         ▼                     ▼

GPIO           USART                  TIMER                ADC

The CPU doesn't need a special instruction called:

TURN_GPIO_ON

Instead, it performs a memory-mapped access to a peripheral register.

For example:

CPU

│

│ write/read address

▼

0x40010800

│

▼

GPIOA peripheral

│

▼

GPIOA register

│

▼

GPIO hardware

│

▼

PAx pin

That is the foundation of bare-metal STM32 programming.

Now let's make it concrete

We know from the official STM32F103 memory map:

GPIOA base = 0x40010800

Suppose we have a GPIOA register with offset:

0x04

Then:

Register address

=

GPIOA base + register offset

=

0x40010800 + 0x04

=

0x40010804

Exactly the calculation you've already been practicing.

And then we can represent that address in C:

volatile uint32_t *REG = (volatile uint32_t *)0x40010804;

Now:

REG

│

└── contains 0x40010804

│

▼

GPIOA

register

And:

*REG |= (1 << 5);

means:

Read register

↓

Set bit 5

↓

Write register back

We have now connected:

C → pointer → address → register → peripheral → hardware.

But we're not going to memorize addresses blindly

This is where professional embedded engineers differ from people who merely memorize code.

I want you to eventually be able to open RM0008 and independently answer:

"Why is GPIOA at 0x40010800?"

and then:

"Why is GPIOA's next register at a particular offset?"

and then:

"What does bit 5 actually control?"

and finally:

"What electrical behavior does that bit cause on PA5?"

That entire chain is our goal.

Checkpoint 0.5.1 🧠

Using the official STM32F103 memory map above:

Q1

What is the base address of:

GPIOA

Q2

What is the base address of:

GPIOB

Q3

What is the base address of:

USART1

Q4

What is the base address of:

RCC

Q5

If:

GPIOA base = 0x40010800

register offset = 0x0C

calculate the register address.

Q6

If:

GPIOA base = 0x40010800

and you create:

volatile uint32_t *REG =

(volatile uint32_t *)0x40010800;

What does:

REG

contain?

And what does:

*REG

represent?

Answers:

- 0x40010800

- 0x40010C00

- 0x40013800

- 0x40021000

- 0x4001080C

- then reg contains the address 0x40010800 and *reg gives the value present in that address