1.2 STM32F103 Memory & Device Organization

What you need to learn

By the end of this section, you should understand:

- STM32F103 memory map

- Flash region

- SRAM region

- System memory

- Peripheral address space

- Vector table location

- How the CPU sees all of these

- How your C program relates to these memory regions

- Basic connection to the linker

We are not going deep into linker scripts yet. That comes later when it becomes useful.

1. The most important idea

From Level 0, you learned:

The CPU sees peripherals through memory-mapped addresses.

Now apply that to the STM32F103.

The Cortex-M3 has a 32-bit address space:

32 bits

↓

0x00000000 to 0xFFFFFFFF

That's a theoretical address space of:

4 GB

But this does not mean the STM32F103 physically contains 4 GB of memory.

It means the CPU has a 32-bit address space in which different regions are assigned different purposes.

Think of it as a huge address map:

0x00000000

│

│

├── Code / memory region

│

├── SRAM region

│

├── Peripheral region

│

├── System memory

│

└── Other reserved/unused regions

│

0xFFFFFFFF

2. STM32F103 memory organization

For the STM32F103 family, the important regions we'll work with are:

STM32F103

│

┌───────────────┼────────────────┐

│                                  │                                    │

Flash                        SRAM                      Peripherals

│                                  │                                    │

Program code        Runtime data           GPIO/RCC/USART/

Vector table                Stack                      Timers/ADC/etc.

Constants                  Variables

There is also System Memory, which contains ST-provided system firmware such as the factory bootloader.

We'll discuss that separately.

3. Flash

For the STM32F103C8T6, the commonly specified Flash capacity is 64 KB.

Its Flash is mapped beginning at:

0x08000000

So conceptually:

0x08000000

│

├── Vector table

├── Program code

├── Constants

└── Other Flash-resident data

For a 64 KB device, the nominal end of that Flash region is:

0x0800FFFF

because:

0x08000000 + 0x10000 - 1

= 0x0800FFFF

Important

Don't confuse:

0x08000000

with:

"The first instruction."

The beginning of the Flash region contains the vector table, whose first entries include the initial stack pointer and reset handler address.

We studied this in Level 0.11.

4. SRAM

The STM32F103C8T6 has 20 KB SRAM.

The SRAM region begins at:

0x20000000

20 KB = 0x5000 bytes.

Therefore, the nominal range is:

0x20000000

↓

0x20004FFF

Conceptually:

SRAM

0x20000000

│

├── Global/static runtime data

├── .bss

├── Stack

├── Local variables

└── Other runtime storage

│

0x20004FFF

The exact placement of these sections is determined by the linker/startup configuration, so don't memorize this drawing as meaning that .bss is always physically first, followed by stack, etc.

The important thing is:

SRAM is the runtime working memory.

5. System Memory

STM32 devices also contain a System Memory region.

This is not the same thing as your user Flash.

ST uses system memory for factory-provided functionality, including the built-in bootloader on supported devices.

Conceptually:

STM32 memory

│

├── User Flash

│

├── SRAM

│

├── Peripheral space

│

└── System Memory

↓

Factory firmware

This becomes relevant later when we discuss:

- boot modes

- bootloader

- firmware programming

- system boot process

For Level 1, that's enough.

6. Peripheral address space

Now we reach something you've already used.

Peripherals are assigned addresses.

For example, for STM32F103:

GPIOA

0x40010800

and:

RCC

0x40021000

So the CPU can perform something conceptually equivalent to:

*(volatile uint32_t *)0x4001080C

and access the corresponding GPIO register.

That's why we call it:

Memory-mapped I/O

The CPU doesn't need a completely separate instruction like:

WRITE_TO_GPIO

Instead, a normal memory access can reach the peripheral through the MCU's bus architecture.

7. Important distinction: physical memory vs address space

This is a common beginner trap.

Suppose we say:

0x40010800 = GPIOA

It does not mean:

GPIOA is SRAM.

It means:

That address belongs to the peripheral address region, and accesses to that address are routed to the GPIO peripheral.

So:

Address

↓

What is mapped there?

↓

Flash / SRAM / peripheral / system memory / etc.

The address tells the CPU what region it is accessing.

8. Where does the vector table fit?

For normal execution from internal Flash, the vector table is located at the beginning of the Flash region:

0x08000000

Conceptually:

0x08000000

┌────────────────────────────┐

│ Initial MSP value                               │

├──────────────────────────┤

│ Reset_Handler address                   │

├──────────────────────────┤

│ NMI_Handler address                     │

├──────────────────────────┤

│ HardFault_Handler address          │

├──────────────────────────┤

│ ...                                                        │

└───────────────────────────┘

This connects three topics we've already studied:

Flash

+

Vector table

+

Startup sequence

9. How does the CPU see all of this?

Here's the mental model I want you to develop:

So when the CPU executes:

int x = 10;

the required memory access is ultimately directed toward an appropriate memory location.

When it executes something that accesses:

GPIOA_ODR

the access goes into the peripheral region instead.

Same CPU.

Same address space.

Different mapped destination.

10. How CubeIDE enters the picture

Suppose you write:

int counter = 10;

int main(void)

{

while (1)

{

counter++;

}

}

Your source code doesn't explicitly say:

"Put counter at this exact SRAM address."

The compiler and linker determine where program/data sections go according to the target and linker configuration.

Conceptually:

C source

↓

Compiler

↓

Object files

↓

Linker

↓

Memory layout

├── Flash → program

└── SRAM  → runtime data

The linker script contains the memory-region definitions used during linking.

You don't need to master linker syntax yet.

For now, remember:

The linker helps place your program's sections into the MCU's memory regions.

11. One subtle but important point

Don't think:

"Every variable goes into SRAM."

That's not correct.

For example:

const int table[] = {1, 2, 3, 4};

may be placed in a read-only section in Flash.

Whereas:

int counter;

is writable runtime data and normally resides in SRAM.

So the compiler/linker classify program objects into sections such as:

.text    → program code

.rodata  → read-only data

.data    → initialized writable data

.bss     → zero-initialized/uninitialized static data

We'll revisit these later.

For now, don't memorize every section. Just understand why memory placement exists.

🧠 The STM32F103 memory picture

For our course, keep this simplified picture:

STM32F103 ADDRESS SPACE

0x00000000

│

│   Code / alias / device-specific mapping

│

├──────────────────────────────

│

│

0x08000000 ──► Internal Flash

│          Program

│          Vector table

│          Constants

│

├──────────────────────────────

│

0x20000000 ──► Internal SRAM

│          Variables

│          Stack

│          Runtime data

│

├──────────────────────────────

│

0x40000000 ──► Peripheral region

│          GPIO

│          RCC

│          USART

│          Timers

│          ADC

│          etc.

│

├──────────────────────────────

│

│          System Memory

│          Other mapped regions

│

0xFFFFFFFF

Important: this is a teaching map, not a complete reproduction of every ARM/STM32 memory-map region. We will use the exact reference manual tables when we need individual regions or registers.

1.2 checkpoint

Just 3 questions:

Q1

For the STM32F103C8T6, where does its internal:

a) Flash start?
b) SRAM start?

Q2

Why can this:

GPIOA_ODR |= (1 << 5);

ultimately access hardware even though the CPU is performing a memory operation?

Q3

If you have:

int counter = 10;

and:

const int values[] = {1, 2, 3};

should you automatically assume both must be stored in SRAM? Why or why not?

Answers:

- 0x08000000 , 0x20000000

- because its memory mapped with peripheral gpioa

- no const variables are stored in flash because they are ro data and normal variables are runtime so they store in sram

Excellent. 3/3 correct. ✅

A small precision on Q3:

Your reasoning is correct, but say "typically" rather than "const variables are stored in Flash" as an absolute rule.

For example:

const int values[] = {1, 2, 3};

is normally placed in a read-only section such as .rodata, which is typically located in Flash.

And:

int counter = 10;

is writable initialized data, typically placed in .data, which resides in SRAM at runtime.

So the better professional wording is:

The compiler/linker determine the section and memory placement. Read-only data is typically placed in Flash, while writable runtime data is typically placed in SRAM.

Your 1.2 understanding

Flash       → 0x08000000

SRAM        → 0x20000000

Peripherals → memory-mapped region

Vector table→ beginning of Flash for normal execution

Linker      → determines placement of program sections

1.2 STM32 Memory & Device Organization: ✅ COMPLETE

We can now move to 1.3 STM32 Documentation, which is actually one of the most important professional habits we'll build.