0.6 Registers & Bit Manipulation: status

1. Registers and addresses ✅

You understand:

uint32_t *REG = (uint32_t *)0x40001000;

and the distinction:

REG   → contains an address

*REG  → accesses the value at that address

You also correctly understood that changing the register's value does not change its address.

2. = vs |= vs &= vs ^= ✅

We've covered:

REG = VALUE;

Replace the entire value.

REG |= MASK;

Set selected bits while preserving the others.

REG &= MASK;

AND operation, commonly used with masks.

REG ^= MASK;

Toggle selected bits.

These are part of the register-programming vocabulary we established.

3. Bit shifting << and >> ✅

You've practiced:

1 << 0  → 0x01

1 << 1  → 0x02

1 << 2  → 0x04

1 << 3  → 0x08

1 << 4  → 0x10

1 << 5  → 0x20

The important mental model is:

1 << N

↓

create a mask for bit N

You've also worked through the difference between:

1 << 2

and:

REG & (1 << 2)

The first creates the mask. The second checks the actual register bit.

4. ~ and clearing bits ✅

You've practiced:

REG &= ~(1 << 5);

Meaning:

Clear bit 5 while preserving the other bits.

And you worked through:

0x20

↓

~0x20

↓

11111111 11111111 11111111 11011111

This is important because:

REG = 0;

would destroy all bits, whereas:

REG &= ~(1 << 5);

targets only bit 5.

5. Bit-field manipulation ✅

We went beyond individual bits and worked with a multi-bit field.

For PA5:

CRL bits 23:20

You learned the professional pattern:

REG = (REG & ~MASK) | VALUE;

For example:

GPIOA_CRL =

(GPIOA_CRL & ~0x00F00000)

| (0x3 << 20);

The underlying idea:

READ

↓

CLEAR field

↓

INSERT new value

↓

WRITE

This is a very important embedded programming pattern.

6. GPIO register application ✅

You've connected bit manipulation to actual STM32 registers:

CRL → configure PA0–PA7

CRH → configure PA8–PA15

ODR → output data

IDR → input data

For PA5:

CRL bits 23:20 → configuration

ODR bit 5      → output

IDR bit 5      → input state

7. RCC bit manipulation ✅

You also applied the same concepts to RCC:

RCC

↓

APB2ENR

↓

IOPAEN = bit 2

and:

RCC_APB2ENR |= (1 << 2);

means:

create bit-2 mask

↓

set bit 2

↓

IOPAEN = 1

↓

GPIOA clock enabled

So you have already started seeing that bit manipulation is not an isolated C exercise. It is how we control real MCU hardware.

So, have we finished 0.6?

Conceptually: YES ✅

Mastery: Not yet 🔶

I don't want to artificially stretch 0.6 into 50 exercises. You've already done enough examples to move forward.

The only thing I want to keep strengthening throughout the entire course is this:

bit number

↓

mask

↓

AND / OR / XOR / NOT

↓

register value

↓

hardware behavior

You'll keep practicing it naturally when we study GPIO, RCC, timers, interrupts, UART, etc.

So I would officially mark:

0.6 Registers & Bit Manipulation    ✅ COMPLETE

and move to:

0.7 ARM Cortex-M Programmer's Model

But before teaching 0.7, I agree that we should give you a quick overview of the remaining Level 0 roadmap so you know exactly where we're going:

0.7  Programmer's Model

R0-R12, SP, LR, PC, xPSR, MSP, PSP

↓

0.8  Buses

AHB, APB, peripheral clock domains

↓

0.9  Clock & Reset

clock source → PLL/prescaler → system/bus/peripheral clocks

↓

0.10 Interrupt Fundamentals

event → IRQ → NVIC → vector table → ISR

↓

0.11 Startup Sequence

RESET → SP → Reset_Handler → startup → main()

↓

0.12 C → Assembly → Hardware

C → compiler → assembly → machine code → CPU → hardware

↓

LEVEL 0 COMPLETE

↓

LEVEL 1 🚀

So yes, 0.6 is done. We can officially move to 0.7 now.